MOD	EUR/13A41/11

TABLE  5-1 (end)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| ... |  |  |  |  |  |
| No. 9.21 Terrestrial, GSO, non-GSO/ terrestrial, GSO, non-GSO | A station of a service for which the requirement to obtain the agreement of other administrations is included in a footnote to the Table of Frequency Allocations referring to No. 9.21 | Band(s) indicated in the relevant footnote | 1) Incompatibility established by the use of Appendices 7, 8, technical Annexes of Appendices 30 or 30A, pfd values specified in some of the footnotes, other technical provisions of the Radio Regulations or ITU-R Recommendations, as appropriate; or 2) In the band 11.7-12.2 GHz (GSO FSS in Region 2): –124 dB(W/(m2 · MHz)) for 0°    5 –124  0.5 (– 5) dB(W/(m2 · MHz)) for 5° <   25 –114 dB(W/(m2 · MHz)) for  > 25 where  is the angle of arrival of the incident wave above the horizontal plane (degrees) | Methods specified in, or adapted from, Appendices 7, 8, 30, 30A, other technical provisions of the Radio Regulations or ITU-R Recommendations |  |

Approach 2