ADD	EUR/13A42/4

DRAFT  Resolution  [EUR/13A42-3]  (WRC-03)

Use of the frequency band 36-37 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the frequency band 36-37 GHz is allocated to the Earth exploration-satellite service (EESS) (passive) and to the space research service (passive) on a primary basis;

b)	that the frequency band 36-37 GHz is allocated to the fixed service and to the mobile service on a primary basis;

c)	that the EESS (passive) protection criteria are contained in Recommendation ITU-R SA.1029;

d)	that Recommendation ITU-R F.758-2 provides characteristics of fixed service point-to-multipoint systems operating in the band 36-37 GHz, but does not provide information on characteristics of fixed service point-to-point systems operating in this band;

e)	that the band 36-37 GHz is not available for high-density applications in the fixed service (see No. 5.547);

f)	that the EESS (passive) operating in the band 36-37 GHz may be interfered by the emissions of systems of active services,

recognizing

1	that EESS (passive) systems may experience harmful interference if a high density of fixed-service stations is deployed in the band 36-37 GHz;

2	that sharing criteria between EESS (passive) and fixed service systems need to be defined in the band 36-37 GHz,

resolves

1	to invite ITU-R to conduct sharing analyses between passive services and the fixed and mobile services in the band 36-37 GHz in order to define appropriate sharing criteria;

2	to recommend that WRC-07 review the results of the studies and consider the inclusion of the sharing criteria within the Radio Regulations,

urges administrations

1	to provide characteristics of active systems (fixed and mobile services) operating in the band 36-37 GHz;

2	to avoid deploying a high density of stations in the fixed and mobile services in the band 36-37 GHz.