| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 5 to Document 13-E |
|  |  |  | 13 March 2003 |
|  |  |  | Original: English/ French/ Spanish |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| european common proposals for the work of the conference |  |
| part 5 |  |

Agenda item 1.5 – Allocations in the frequency range 5 150-5 725 MHz

Introduction

WRC-03 agenda item 1.5 invites the Conference “to consider in accordance with Resolution 736 (WRC-2000), regulatory provisions and spectrum requirements for new and additional allocations to the mobile, fixed, Earth exploration-satellite and space research services, and to review the status of the radiolocation service in the frequency range 5 150-5 725 MHz, with a view to upgrading it, taking into account the results of ITU-R studies”. Based on the assessment requested by Resolution 736 (WRC-2000), Europe proposes that:

1	The bands 5 150-5 350 MHz and 5 470-5 725 MHz be allocated worldwide on a primary basis to the mobile service for the implementation of wireless access systems (WAS) including RLANs, ensuring non-interference with other services by footnotes 5.XXX and 5.YYY specifying the relevant ITU-R Recommendations with appropriate operational restrictions and e.i.r.p. limits.

2	The allocation to the fixed service in Region 3 in the band 5 250-5 350 MHz for the implementation of fixed wireless access (FWA) systems ensure the protection of the Earth exploration-satellite (active) and space research services in this band. The allocation to the fixed service in Region 3 should not prevent the worldwide primary allocation to the mobile service as proposed under point 1.

3	The additional primary allocation to EESS (active) in the frequency range 5 460-5 570 MHz be subject to no additional constraints to the worldwide primary allocation to the mobile service as proposed under point 1.

4	The radiolocation service in the frequency range 5 350-5 650 MHz be upgraded. In the band 5 470-5 650 MHz the upgrade is also seen as a consequence to the worldwide primary allocation to the mobile service as proposed under point 1.

Proposals

ARTICLE  5

MOD	EUR/13A5/1

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 150-5 250 AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE ADD 5.XXX ADD 5.YYY 5.446 5.447B 5.447C |  |  |

MOD	EUR/13A5/2

5 250-5 255 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.447D MOBILE ADD 5.XXX ADD 5.YYY 5.448 5.448A

5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) MOBILE ADD 5.XXX ADD 5.YYY 5.448 5.448A

MOD	EUR/13A5/3

5 350-5 460 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B AERONAUTICAL RADIONAVIGATION 5.449 RADIOLOCATION

MOD	EUR/13A5/4

5 460-5 470 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B RADIONAVIGATION 5.449 RADIOLOCATION

MOD	EUR/13A5/5

5 470-5 570 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B MARITIME RADIONAVIGATION MOBILE ADD 5.XXX ADD 5.YYY RADIOLOCATION 5.450 5.451

MOD	EUR/13A5/6

5 570-5 650 MARITIME RADIONAVIGATION MOBILE ADD 5.XXX ADD 5.YYY RADIOLOCATION 5.450 5.451 5.452

MOD	EUR/13A5/7

5 650-5 725 MOBILE ADD 5.XXX ADD 5.YYY RADIOLOCATION Amateur Space research (deep space) 5.282 5.451 5.453 5.454 5.455

Reasons:

•	With the global growth in the mobile Internet and multimedia applications the use of wireless access systems including RLANs (WAS/RLANs) has enormous potential worldwide which requires harmonized additional spectrum.

•	The proposed changes within the allocation table are interrelated.

•	The global allocation of the bands 5 150-5 350 MHz and 5 470-5 725 MHz to the mobile service on a primary basis for the implementation of wireless access systems including RLANs (WAS/RLANs) will provide an appropriate regulatory status for their operation and development.

•	WAS/RLANs will offer many operational and economic benefits to users in commercial, educational and leisure activities as well as to countries with developing communications infrastructures.

•	The allocation of the whole frequency range 5 250-5 570 MHz to the EESS (active) will allow the use of wideband sensors providing an increased resolution.

•	The upgrade of the radiolocation service in the band 5 350-5 650 MHz will provide the same regulatory protection to it over the whole frequency range. The upgrade of the radiolocation service in the band 5 470-5 650 MHz is also a consequence of EUR/13A5/9. Within the frequency range 5 460-5 570 MHz the upgrade of the RLS is also seen as a consequence to the proposed worldwide allocation to the EESS (active).

SUP	EUR/13A5/8

5.447

Reasons:	Suppression of No. 5.447 is conditional to the primary allocation of the band 5 150-5 250 MHz to the mobile service as proposed in EUR/13A5/1.

ADD	EUR/13A5/9

5.XXX	The use of the bands 5 150-5 350 MHz and 5 470-5 725 MHz by the mobile service is for the implementation of wireless access systems, including RLANs. Stations in the mobile service shall be operated in accordance with the conditions below:

- In the 5 150-5 350 MHz band, the use of stations shall be restricted to indoor use with a maximum mean e.i.r.p. of 200 mW (averaged over the transmission burst at the highest power setting), with a mean e.i.r.p. density not exceeding 0.04 mW/4 kHz in any 4 kHz bandwidth.

- 	In the band 5 470-5 725 MHz, the use of stations shall be restricted to a maximum mean e.i.r.p. of 1 W (averaged over the transmission burst at the highest power setting), with a mean e.i.r.p. density not exceeding 50 mW/1 MHz in any 1 MHz bandwidth.

- 	Equipment operating in the bands 5 250-5 350 MHz and/or 5 470-5 725 MHz, shall employ transmitter power control to provide a mitigation factor of at least 3 dB on the maximum mean output power. If transmitter power control is not in use, the maximum mean e.i.r.p. shall be reduced by 3 dB.

- Equipment operating in the bands 5 250-5 350 and/or 5 470-5 725 MHz shall use dynamic frequency selection as referred to in Resolution [EUR/13A5-5 GHz] (WRC-03) to provide protection to co-primary terrestrial services. The stations shall also be designed to provide, on aggregate, a uniform spread of the loading of the stations across the available spectrum to improve sharing with satellite services.

Reasons:

•	The conditions and mitigation techniques described within footnote 5.XXX are required to protect MSS feeder links in the band 5 150-5 250 MHz allocated to FSS (Earth-to-space), the EESS (active) in the band 5 250-5 350 MHz and the radiodetermination service in the bands 5 250-5 350 MHz/5 470-5 725 MHz while allowing the operation of WAS/RLANs.

•	Frequency sharing based on active avoidance of co-channel operation with existing systems (by DFS) requires an adequate amount of spectrum for WAS/RLANs.

•	The content of proposed footnote 5.XXX is in line with the content of footnote 5.XX, proposed with respect to agenda item 1.6 of WRC-03 (see EUR/13A6/2).

ADD	EUR/13A5/10

5.YYY	In the band 5 150-5 250 MHz stations in the mobile service shall not claim protection from earth stations in the fixed-satellite service. In the bands 5 250-5 350 MHz and 5 470-5 725 MHz stations in the mobile service shall not claim protection from the radiodetermination service. The provisions of No. 5.43A do not apply to the mobile service allocations in the bands 5 150-5 350 MHz and 5 470-5 725 MHz.

MOD	EUR/13A5/11

5.448B	The earth exploration-satellite (active) service operating in the band 5 350-5 570 MHz shall not cause harmful interference to, or constrain the use and development of, the aeronautical radionavigation service in the band 5 350-5 460 MHz, the radionavigation service in the band 5 460-5 470 MHz and the maritime radionavigation in the band 5 470-5 570 MHz.   (WRC-03)

Reasons:	This footnote will also be applied to the band 5 460-5 570 MHz as a consequence of the proposed allocation to the EESS (active).

ADD	EUR/13A5/12

DRAFT  RESOLUTION  [EUR/13A5-5 GHz]  (WRC-03)

Characteristics of dynamic frequency selection to be used by stations in the mobile service operating in the bands 5 250-5 350 MHz and 
5 470-5 725 MHz to protect the radiodetermination service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that this conference allocated the bands 5 150-5 350 MHz and 5 470-5 725 MHz on a primary basis to the mobile service for the implementation of wireless access systems (WAS), including radio local area networks (RLANs);

b)	the need to protect existing and planned radiodetermination systems in the bands 5 250-5 350 MHz and 5 470-5 725 MHz;

c)	that studies have shown that sharing between these services is only possible with the application of mitigation techniques such as dynamic frequency selection (DFS),

resolves

that, in order to ensure adequate protection to the radiodetermination systems, stations in the mobile service operating in the 5 250-5 350 MHz and 5 470-5 725 MHz bands shall implement the mitigation measures contained in Annex 1 of Recommendation ITU-R M.[8/152+Corr.1],

invites ITU-R

to conduct, as a matter of urgency and taking account of practical experience, further studies on the suitability of the DFS characteristics as referred to in resolves.

_________________