ADD	EUR/13A5/12

DRAFT  RESOLUTION  [EUR/13A5-5 GHz]  (WRC-03)

Characteristics of dynamic frequency selection to be used by stations in the mobile service operating in the bands 5 250-5 350 MHz and 
5 470-5 725 MHz to protect the radiodetermination service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that this conference allocated the bands 5 150-5 350 MHz and 5 470-5 725 MHz on a primary basis to the mobile service for the implementation of wireless access systems (WAS), including radio local area networks (RLANs);

b)	the need to protect existing and planned radiodetermination systems in the bands 5 250-5 350 MHz and 5 470-5 725 MHz;

c)	that studies have shown that sharing between these services is only possible with the application of mitigation techniques such as dynamic frequency selection (DFS),

resolves

that, in order to ensure adequate protection to the radiodetermination systems, stations in the mobile service operating in the 5 250-5 350 MHz and 5 470-5 725 MHz bands shall implement the mitigation measures contained in Annex 1 of Recommendation ITU-R M.[8/152+Corr.1],

invites ITU-R

to conduct, as a matter of urgency and taking account of practical experience, further studies on the suitability of the DFS characteristics as referred to in resolves.