ADD	EUR/13A5/9

5.XXX	The use of the bands 5 150-5 350 MHz and 5 470-5 725 MHz by the mobile service is for the implementation of wireless access systems, including RLANs. Stations in the mobile service shall be operated in accordance with the conditions below:

- In the 5 150-5 350 MHz band, the use of stations shall be restricted to indoor use with a maximum mean e.i.r.p. of 200 mW (averaged over the transmission burst at the highest power setting), with a mean e.i.r.p. density not exceeding 0.04 mW/4 kHz in any 4 kHz bandwidth.

- 	In the band 5 470-5 725 MHz, the use of stations shall be restricted to a maximum mean e.i.r.p. of 1 W (averaged over the transmission burst at the highest power setting), with a mean e.i.r.p. density not exceeding 50 mW/1 MHz in any 1 MHz bandwidth.

- 	Equipment operating in the bands 5 250-5 350 MHz and/or 5 470-5 725 MHz, shall employ transmitter power control to provide a mitigation factor of at least 3 dB on the maximum mean output power. If transmitter power control is not in use, the maximum mean e.i.r.p. shall be reduced by 3 dB.

- Equipment operating in the bands 5 250-5 350 and/or 5 470-5 725 MHz shall use dynamic frequency selection as referred to in Resolution [EUR/13A5-5 GHz] (WRC-03) to provide protection to co-primary terrestrial services. The stations shall also be designed to provide, on aggregate, a uniform spread of the loading of the stations across the available spectrum to improve sharing with satellite services.

Reasons:

•	The conditions and mitigation techniques described within footnote 5.XXX are required to protect MSS feeder links in the band 5 150-5 250 MHz allocated to FSS (Earth-to-space), the EESS (active) in the band 5 250-5 350 MHz and the radiodetermination service in the bands 5 250-5 350 MHz/5 470-5 725 MHz while allowing the operation of WAS/RLANs.

•	Frequency sharing based on active avoidance of co-channel operation with existing systems (by DFS) requires an adequate amount of spectrum for WAS/RLANs.

•	The content of proposed footnote 5.XXX is in line with the content of footnote 5.XX, proposed with respect to agenda item 1.6 of WRC-03 (see EUR/13A6/2).