ADD	ASP/25/116

DRAFT  RESOLUTION  [AMSS]  (WRC-03)

Use of the aeronautical mobile-satellite service (Earth-to-space) 
in the band 14-14.5 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the band 14-14.5 GHz was allocated to the land mobile-satellite service (Earth-to-space) on a secondary basis prior to WRC-97;

b)	that the services also allocated in the 14-14.5 GHz band, fixed-satellite (Earth-to-space), radionavigation, fixed and mobile, need to be protected consistent with their allocation status;

c)	that there is a demand for use on board aircraft of aeronautical mobile-satellite service capabilities in order to provide two-way communication and data transmission functions;

d)	that studies carried out by ITU-R as contained in Recommendation ITU-R M.[AMSS] indicated that the use of the band by AMSS is feasible;

e)	that this Conference has broadened the allocation in the 14-14.5 GHz band to include aeronautical mobile-satellite services,

recognizing

that under current procedures, in order for notification to proceed, the class of station and category of allocation of the AMSS aircraft earth station (AES) need to be matched with those of the fixed-satellite service (FSS) assignment under which that AES operates or is planned to be operated,

resolves

1	that in order to use an FSS transponder with primary status as an AMSS transponder on a secondary basis together with an AES on board aircraft on secondary status in the frequency band 14-14.5 GHz;

1.1	that the class of station and category of allocation of that FSS assignment shall be modified from FSS to AMSS and primary to secondary respectively, in relation to its use for the AMSS to match the class of station and category of allocation of both FSS transponder and AES with each other;

1.2	that, upon first notification by an administration responsible for an AMSS system, the characteristics of the FSS assignment referred to in resolves 1.1 above be published in a BR IFIC together with the characteristics of the AES referred to in the same resolves;

2	that the characteristics of any AMSS earth station published in accordance with resolves 1.2 above shall remain within the envelope of characteristics of the earth stations published by the Radiocommunication Bureau in the Special Section pertaining to the FSS specific or typical network system;

3	that, in the case where the conditions prescribed in resolves 2 above are not met, the relevant procedures of Articles 9 and 11 of the RR together with their associated Rule of Procedure shall apply,

further resolves

that the course of action referred to in resolves 1, 1.1, 1.2 and 2 above shall preserve the status and rights previously attributed to the FSS assignment (with respect to its category of allocation, class of station and coordination agreement obtained under applicable provisions of the RR) as long as the coordination agreements, together with the associated terms and conditions, are fully respected.

Reasons:	This will enable the matching of the class of station and category of service of the AES with that of the FSS space station and thus facilitate the notification and processing of the AES with ITU.