| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 13 to Document 25-E |
|  |  |  | 15 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Asia-Pacific Common Proposals |  |
| proposals for the work of the conference |  |
| agenda item 1.13 |  |

1.13	to consider regulatory provisions and possible identification of existing frequency allocations for services which may be used by high altitude platform stations, taking into account No. 5.543A and the results of the ITU-R studies conducted in accordance with Resolutions 122 (Rev.WRC-2000) and 734 (WRC-2000)

Introduction

The APT Members propose the modification to No. 5.543A in order to protect the passive services in the adjacent band from systems using HAPS, and the modifications to Resolutions 122 (Rev.WRC-2000) and 734 (WRC-2000) to facilitate further studies.

Proposals

ARTICLE  5

MOD	ASP/25/130

5.543A	In Bhutan, Indonesia, Iran (Islamic Republic of), Japan, Maldives, Mongolia, Myanmar, Pakistan, the Dem. People’s Rep. of Korea, Sri Lanka, Thailand and Viet Nam, the allocation to the fixed service in the band 31-31.3 GHz may also be used by systems using high altitude platform stations (HAPS) in the ground-to-HAPS direction. The use of the band 31-31.3 GHz by systems using HAPS shall not cause harmful interference to, nor claim protection from, other types of fixed-service systems or other co-primary services, taking into account No. 5.545. Systems using HAPS in the band 31-31.3 GHz shall not cause harmful interference to the radio astronomy service having a primary allocation in the band 31.3-31.8 GHz, taking into account the protection criterion of 
–141 dB(W/(m2 · 500 MHz)). In order to ensure the protection of satellite passive services, the level of unwanted emissions from a ground station of a system using HAPS into the band 31.3-31.8 GHz

shall be limited to –106 dB(W/MHz) under a clear-sky condition, and may be increased up to 
–100 dB(W/MHz) under rainy conditions in accordance with rain attenuation, provided that effective impact on the passive services is not in excess of –106 dB(W/MHz).     (WRC-03)

Reasons:	For the operation of systems using HAPS in the FS in the band 31.0-31.3 GHz, the proposed modification of No. 5.543A is suitable for the protection of passive services in the 31.3-31.8 GHz band, taking into account the result of ITU-R studies.

MOD	ASP/25/131

RESOLUTION  122  (Rev.WRC-03)

Use of the bands 47.2-47.5 GHz and 47.9-48.2 GHz by high altitude platform stations (HAPS) in the fixed service and by other services and the potential use of the bands 27.5-28.35 GHz and 31.0-31.3 GHz by HAPS in the fixed service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the band 47.2-50.2 GHz is allocated to the fixed, mobile and fixed-satellite services on a co-primary basis;

b)	that WRC-97 made provision for operation of HAPS, also known as stratospheric repeaters, within the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz;

c)	that ITU has among its purposes “to promote the extension of the benefits of the new telecommunication technologies to all the world’s inhabitants” (No. 6 of the Constitution);

d)	that systems based on new technologies using high altitude platforms will be able to provide high-capacity, competitive services to urban and rural areas;

e)	that the development of any service requires major investment and that manufacturers and operators should be given the confidence to make the necessary investment;

f)	that high altitude platform systems are in an advanced stage of development and some countries have notified such systems to ITU in the bands 47.2-47.5 GHz and 47.9-48.2 GHz;

g)	that WRC-97 adopted a definition of HAPS in Article 1, modified No. 11.24 and added No. 11.26 providing for notices relating to assignments for HAPS in the bands 47.2-47.5 GHz and 47.9-48.2 GHz and that the Radio Regulations Board issued a provisional rule of procedure concerning notification periods in No. 11.24/1228 in February 1997;

h)	that while the decision to deploy HAPS can be taken on a national basis, such deployment may affect neighbouring administrations, particularly in small countries;

i)	that ITU-R has completed studies dealing with sharing between systems using HAPS in the fixed service and other types of systems in the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz;

j)	that the radio astronomy service has primary allocations in the bands 42.5-43.5 GHz and 48.94-49.04 GHz;

k)	that No. 5.552 urges administrations to reserve fixed-satellite service (FSS) use of the band 47.2-49.2 GHz for feeder links for the broadcasting-satellite service, and that ITU-R studies indicate that HAPS in the fixed service may share with broadcasting-satellite feeder links, providing that there is adequate geographical separation between HAPS and the feeder links concerned;

l)	that Recommendation ITU-R SF.1481-1 provides useful reference information in regard to sharing between systems using HAPS and GSO FSS but also notes that further study of operational scenarios and mitigation techniques is required, and such study would enable greater confidence in sharing the radio spectrum in the 47.2-47.5 GHz band and 47.9-48.2 GHz band, which have been designated for systems using HAPS;

m)	that since the 47 GHz bands are more susceptible to rain attenuation in certain areas of Region 3, the range 18-32 GHz has been proposed for Region 3 for possible identification of additional spectrum in ITU-R, and ITU-R studies are in progress for these bands;

n)	that the 18-32 GHz range is already heavily used by a number of different services and a number of other types of applications in the fixed service;

o)	that Nos. 5.537A and 5.543A permit the use of HAPS in the fixed service in the bands 27.5-28.35 GHz and 31.0-31.3 GHz in certain countries on a non-harmful interference, non-protection basis in order to address issues of rain attenuation associated with the 47 GHz bands referred to in considering b) above;

p)	that ITU-R has conducted studies dealing with sharing between systems using HAPS in the fixed service and other types of systems in the fixed service in the bands 27.5-28.35 GHz and 31.0-31.3 GHz leading to Recommendation ITU-R F.1609;

q)	that ITU-R has conducted studies dealing with compatibility between systems using HAPS and the passive service in the 31.3-31.8 GHz band, leading to Recommendation ITU-R F.1570-1 and Recommendation ITU-R F.1612;

r)	that ITU-R has produced draft new Recommendation ITU-R SF.1601 in regard to interference evaluation methodology from the fixed-service system using HAPS into GSO FSS in the band 27.5-28.35 GHz to facilitate further studies;

s)	that technical, sharing and regulatory issues should be studied in order to determine criteria for the operation of HAPS in the bands referred to in considering l) and r) above;

t)	that the 31.3-31.8 GHz band is allocated to the radio astronomy, Earth exploration-satellite (passive) and space research (passive) services and the 31.8-32.3 GHz band is allocated to the space research (deep space) service, and that there is a need to appropriately protect these

services from unwanted emissions, taking into account No. 5.340 and the interference criteria given in Recommendations ITU-R SA.1029 and ITU-R RA.769;

u)	that results of ITU-R studies have been presented which indicate that in the bands at 47.2-47.5 GHz, 47.9-48.2 GHz, 27.5-28.35 GHz and 31.0-31.3 GHz sharing between fixed-service systems using HAPS and other conventional fixed-service systems in the same area will require appropriate interference mitigation techniques to be developed and implemented,

resolves

1	to urge administrations to facilitate coordination between HAPS in the fixed service operating in the bands 47.2-47.5 GHz and 47.9-48.2 GHz and other co-primary services in their territory and adjacent territories;

2	that, on a provisional basis, the procedures of Article 9 shall be used for coordination between satellite systems and systems using HAPS in the bands 47.2-47.5 GHz and 47.9-48.2 GHz;

3	that, on a provisional basis, the administrations which intend to implement systems using HAPS in the fixed service in the bands 27.5-28.35 GHz and 31.0-31.3 GHz shall seek agreements of neighbouring administrations with regard to their primary services to ensure that conditions in Nos. 5.537A and 5.543A are met, and the information on the HAPS implementation shall be sent to the Bureau;

4	to invite WRC-07 to review the results of the studies specified below and consider refinement of the regulatory provisions that might facilitate a broader application of these high altitude platform technologies,

requests ITU-R

1	to study the regulatory provisions that might be needed in order to address those cases where the deployment of HAPS in the territory of one administration may affect neighbouring administrations;

2	to continue to carry out studies on the appropriate technical sharing criteria for the situations referred to in considering l) and u) above, in a most efficient and harmonized manner with respect to the operational environments and the requirements of the systems in FSS;

--

3	to study the interference criteria on the basis of Nos. 5.537A and 5.543A in the bands 27.5-28.35 GHz and 31.0-31.3 GHz,

instructs the Director of the Radiocommunication Bureau

1	that notices concerning HAPS that were received by the Bureau prior to 22 November 1997, and provisionally recorded in the Master International Frequency Register in accordance with the provisional rule of procedure issued by the Board, shall be maintained;

2	that from 22 November 1997, and pending review of the sharing studies in considering l) and review of the notification process by WRC-07, the Bureau shall accept notices in the bands 47.2-47.5 GHz and 47.9-48.2 GHz only for HAPS in the fixed service and for feeder links for the broadcasting-satellite service, shall continue to process notices for FSS networks (except for feeder links for the broadcasting-satellite service) for which complete information for

advance publication has been received prior to 27 October 1997, and shall inform the notifying administrations accordingly;

3	that the Bureau shall publish the information on the HAPS implementation received from the administration which intends to implement a system using HAPS in the fixed service in the bands 27.5-28.35 GHz and 31.0-31.3 GHz.

Reasons:	It is recognized that studies under Resolution 122 are incomplete and need to be continued in order to facilitate the introduction of HAPS.

MOD	ASP/25/132

RESOLUTION  734  (Rev.WRC-03)

Feasibility of use by high altitude platform stations in the fixed and mobile services in the frequency bands above 3 GHz allocated exclusively 
for terrestrial radiocommunication

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that ITU has among its purposes “to promote the extension of the benefit of the new telecommunication technologies to all the world’s inhabitants” (No. 6 of the Constitution);

b)	that systems based on new technologies using high altitude platform stations (HAPS) can potentially be used for various applications such as the provision of high-capacity services to urban and rural areas;

c)	that WRC-97 made provision for the use of HAPS within the fixed service in the bands 47.2-47.5 GHz and 47.9-48.2 GHz (see also Resolution 122 [(Rev.WRC-03)]);

d)	that in view of the altitude at which HAPS are placed, the area visible from a HAPS may be within a country or also include neighbouring countries;

e)	that some administrations intend to operate systems using HAPS in the bands allocated exclusively by the Table of Frequency Allocations or by footnotes for terrestrial radiocommunication such as the fixed and mobile services,

recognizing

ITU-R studies relating to geometrical coordination distance for the visible distance from HAPS, as described in Recommendation ITU-R F.1501,

resolves

to recommend to WRC-07 to review the feasibility of facilitating the implementation of systems using HAPS in the fixed and mobile services in bands above 3 GHz allocated exclusively by the Table of Frequency Allocations or by footnotes for terrestrial radiocommunication,

invites ITU-R

to carry out, as a matter of urgency, regulatory and technical studies to determine the feasibility of facilitating systems using HAPS in the fixed and mobile services in bands above 3 GHz allocated exclusively by the Table of Frequency Allocations or by footnotes for terrestrial radiocommunication, taking account of existing use and future requirements in these bands, and any impact on allocations in adjacent bands,

encourages administrations

to contribute actively to the sharing studies in accordance with this Resolution.

Reasons:	Further studies are required on the identification of the feasible frequency bands and necessary regulatory procedures of the systems using HAPS for the fixed and mobile services in the bands above 3 GHz, in accordance with revision of Resolution 734.

_______________