ADD	ASP/25/141

Section VI    Protection of aeronautical radionavigation service systems from aggregate emissions of space stations of radionavigation-satellite service 
systems in the 1 164-1 215 MHz band

21.18	§ 7	Administrations operating or planning to operate radionavigation-satellite service systems or networks in the 1 164-1 215 MHz frequency band, for which complete coordination or notification information, as appropriate, was received by the Bureau after 2 June 2000, shall, in accordance with resolves 2 of Resolution [RNSS 1.2.1-1] (WRC-03), take all necessary steps to ensure that the actual aggregate interference into aeronautical radionavigation service systems caused by such RNSS systems or networks operating co-frequency in these frequency bands does not exceed the equivalent power flux-density level shown in resolves 1 of Resolution [RNSS 1.2.1-1] (WRC-03).

Reasons:	Article 21 of the Radio Regulations addresses sharing between terrestrial and space services in frequency bands above 1 GHz. Placement of this provision is a new Section VI of Article 21 brings into the Radio Regulations the critical elements from new Resolution [RNSS 1.2.1-1] (WRC-03) that makes mandatory the collective obligation of administrations operating RNSS systems at 1 164-1 215 MHz to ensure that the aggregate protection criteria from resolves 1 of Resolution [RNSS 1.2.1-1] (WRC-03) is not exceeded, as well as the requirement to reduce emissions if administrations operating ARNS systems identify excess emission levels.