MOD	ASP/25/27

Methodology and data

The software should use:

–	Recommendation ITU-R BS.705 for the calculation of antenna patterns;

–	Recommendation ITU-R P.533 for the prediction of the wanted field strength values at each test point for each wanted requirement;

–	Recommendation ITU-R P.533 for the prediction of the potentially interfering field-strength values from all other co-channel or adjacent channel requirements at each test point for each wanted requirement;

–	Recommendations 517 (HFBC-87) and ITU-R BS.560 for adjacent channel RF protection ratios;

–	Recommendation ITU-R P.842 for the calculation of reliability values.

The set of 911 test points (agreed at WARC HFBC-87) should be used, supplemented where necessary with test points based on a geographic grid.

The software should calculate the wanted and unwanted field-strength values and the fading margins at each test point inside the required service area.

The desired RF signal-to-noise and RF protection ratios should be user selectable, the default values being 34 dB and 17 dB (DSB-to-DSB co-channel case), respectively. In the case of digital emissions, the corresponding values are as provided in the current ITU-R Recommendation1. The values for RF protection ratios should be used by the Bureau for its compatibility analyses.

...

Reasons:	(Proposals ASP/25/24-27): To enable BR to process digitally modulated requirements under the procedures of Article 12.

1	 	Draft new Recommendation ITU-R BS.[Doc. 6/324(Rev.1)].