MOD	ASP/25/160

5.503	In the band 13.75-14 GHz, geostationary space stations in the space research service for which information for advance publication has been received by the Bureau prior to 31 January 1992 shall operate on an equal basis with stations in the fixed-satellite service; after that date, new geostationary space stations in the space research service will operate on a secondary basis. Until those geostationary space stations in the space research service for which information for advance publication has been received by the Bureau prior to 31 January 1992 cease to operate in this band:

–	the e.i.r.p. density of emissions from any earth station in the fixed-satellite service operating with a space station in geostationary-satellite orbit shall not exceed 4.7D + 28 dB(W/40 kHz), where D is the fixed-satellite service earth station antenna diameter (m) from 13.770 to 13.780 GHz for earth station diameters equal to or greater than 1.2 m and less than 4.5 m;

–	49.2 + 20 log(D/4.5) dB(W/40 kHz), where D is the fixed-satellite service earth station antenna diameter (m) from 13.770 to 13.780 GHz for earth station diameters equal to or greater than 4.5 m and less than 31.9 m;

–	66.2 dB(W/40 kHz) for any fixed-satellite service earth station emission in the band 13.770-13.780 GHz for antenna diameters (m) equal to or greater than 31.9 m;

–	for any fixed-satellite service earth station antenna diameter (m) a maximum e.i.r.p. spectral density of 56.2 dB(W/4 kHz) for narrow-band (less than 40 kHz of necessary bandwidth) fixed-satellite service earth station emissions in the band 13.770-13.780 GHz;

–	the e.i.r.p. density of emissions from any earth station in the fixed-satellite service operating with a space station in non-geostationary-satellite orbit shall not exceed 51 dBW in the 6 MHz band from 13.772 to 13.778 GHz.

Automatic power control may be used to increase the e.i.r.p. density in these frequency range to compensate for rain attenuation, to the extent that the power flux-density at the fixed-satellite service space station does not exceed the value resulting from use by an earth station of an e.i.r.p. meeting the above limits, in clear-sky conditions.     (WRC-03)

Reasons:	To maintain the current protection of SRS operations and to extend the protected bandwidth to 10 MHz.