ADD	ASP/25/162

21.13bis	In the band 13.75-14 GHz, the level of e.i.r.p. emitted by an earth station of a GSO FSS network shall not exceed the following values6bis:

| Angle off-axis | Maximum e.i.r.p. in any 1 MHz band |
| --- | --- |
| 2   7 7   9.2 9.2   48  48 | 43 – 25 log  dBW 22 dBW 46 – 25 log  dBW 4 dBW |

These limits do not apply to FSS earth stations brought into service prior to WRC-03.