| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 26 to Document 25-E |
|  |  |  | 15 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Asia-Pacific Common Proposals |  |
| proposals for the work of the conference |  |
| agenda item 1.26 |  |

1.26	to consider the provisions under which earth stations located on board vessels could operate in fixed-satellite service networks, taking into account the ITU-R studies in response to Resolution 82 (WRC-2000)

Introduction

The APT believes that the introduction of ESVs is appropriate based on the results of ITU-R studies. However, it is important to adequately protect terrestrial services without imposing undue constraints on ESVs for improving the efficiency of telecommunication services and increasing their usefulness.

According to the above general principle, the APT has considered the CPM Report and relevant ITU-R Recommendations and concludes that Method A and the related regulatory and procedural considerations in the CPM Report can be supported.

On this basis, the APT submits the following common proposals.

Proposals

ARTICLE  1

Terms and definitions

NOC	ASP/25/175

Section IV  –  Radio stations and systems

Reasons:	The APT considers that earth stations in the fixed-satellite service may be operated on board vessels, operating in accordance with the specific operational limitation, such as in Resolution [ESV-APT], therefore there is no need to define ESV in Article 1.

ARTICLE  5

MOD	ASP/25/176

5 830-7 550 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 925-6 700 FIXED FIXED-SATELLITE (Earth-to-space) ADD 5.ESV-FSS3 MOBILE 5.149 5.440 5.458 |  |  |

MOD	ASP/25/176A

11.7-14.25 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14-14.25 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV-FSS3 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Space research 5.505 |  |  |

MOD	ASP/25/176B

14.25-15.63 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14.25-14.3 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV-FSS3 RADIONAVIGATION 5.504 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Space research 5.505 5.508 5.509 |  |  |
| 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV-FSS3 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Radionavigation-satellite | 14.3-14.4 FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV-FSS3 Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Radionavigation-satellite | 14.3-14.4 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV-FSS3 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Radionavigation-satellite |
| 14.4-14.47 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV-FSS3 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Space research (space-to-Earth) |  |  |
| 14.47-14.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.484A 5.506 ADD 5.ESV-FSS3 MOBILE except aeronautical mobile Mobile-satellite (Earth-to-space) except aeronautical mobile-satellite Radio astronomy 5.149 |  |  |

ADD	ASP/25/176C

5.ESV-FSS3	In the bands 5 925-6 425 MHz and 14.0-14.5 GHz, earth stations in the fixed-satellite service may be operated on board vessels. Such use shall be in accordance with Resolution [ESV-APT] (WRC-03). Such use shall not cause harmful interference to, claim protection from, or otherwise impose constraints on the operation or development of other radio services operating in the bands 5 925-6 425 MHz and 14.0-14.5 GHz.

Reasons:	The APT considers that earth stations located on board vessels could operate in fixed-satellite service networks in the bands 5 925-6 425 MHz and 14.0-14.5 GHz. Therefore no additional allocation is necessary. Furthermore, the current frequency allocations for MSS on 14.0-14.5 GHz band in Article 5 should not be changed since it is not in the scope of this agenda.

The APT supports Method A and one of the examples of footnote indicated in the CPM Report in the bands 5 925-6 425 MHz and 14.0-14.5 GHz.

(Proposal number ASP/25/177 not used.)

ASP/25/178

With respect to Article 9 (Procedure for effecting coordination with or obtaining agreement of other administrations) and Article 11 (Notification and recording of frequency assignments), the APT considers that earth stations in the fixed-satellite service may be operated on board vessels, operating in accordance with the specific operational limitation, such as in Resolution [ESV-APT] (WRC-03), and the current coordination procedures adequately cover the procedures for ESVs, therefore there is no need to modify the coordination and notification procedure in Articles 9 and 11.

ADD	ASP/25/179

DRAFT  RESOLUTION  [ESV-APT]  (WRC-03)

Provisions relating to earth stations located on board vessels 
which operate in fixed-satellite service networks in the 
uplink bands 5 925-6 425 MHz and 14-14.5 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that there is a demand for global wideband satellite communication services on vessels;

b)	that the technology exists that enables earth stations on board vessels (ESVs) to use fixed-satellite service (FSS) networks operating in the uplink bands 5 925-6 425 MHz and 14-14.5 GHz;

c)	that ESVs have the potential to cause unacceptable interference to other services in the bands 5 925-6 425 MHz and 14-14.5 GHz;

d)	that with respect to the bands considered in this Resolution, global coverage is only available in the band 5 925-6 425 MHz and that only a limited number of geostationary FSS systems can provide such global coverage;

e)	that without special regulatory provisions ESVs could place a heavy coordination burden on some administrations, especially those in developing countries;

f)	that, in order to ensure the protection and future growth of other services, ESVs shall operate with requisite technical and operational constraints;

g)	that, based on agreed technical assumptions, minimum distances from the reference point have been calculated, within ITU-R studies, beyond which an ESV will not have the potential to cause unacceptable interference to other services in the bands 5 925-6 425 MHz and 14-14.5 GHz,

noting

that the regulatory procedures of Article 9 apply for ESVs operating at specified fixed points,

resolves

that ESVs transmitting in the 5 925-6 425 MHz and 14-14.5 GHz bands shall operate under the regulatory and operational provisions contained in Annex 1 and the technical constraints in Annex 2 of this Resolution,

encourages concerned administrations

to cooperate with administrations which license ESVs while seeking agreement under these provisions,

instructs the Secretary-General

to bring this Resolution to the attention of the Secretary-General of the International Maritime Organization (IMO).

ANNEX  1  TO  draft  RESOLUTION  [ESV-APT]  (wrc-03)

Regulatory and operational provisions for ESVs transmitting in the
5 925-6 425 MHz and 14-14.5 GHz bands

1	The administration that issues the licence for the use of ESVs in these bands (licensing administration) shall ensure that such stations follow the provisions of this attachment and thus do not present any potential to cause unacceptable interference to the services of other concerned administrations.

2	ESV service providers shall comply with the technical limitations listed in Annex 2 and, when operating within the minimum distances as identified in § 4 below, with the additional limitations agreed by the licensing and other concerned administrations.

3	In the associated downlink bands, 3 400-4 200 MHz and 10.7-12.75 GHz, ESVs in motion shall not claim protection from transmissions of other services operating in accordance with the Radio Regulations.

4	The minimum distance from the reference point2 beyond which ESVs can operate without the prior agreement of any administration is 300 km in the 5 925-6 425 MHz band and 125 km in the 14-14.5 GHz band. Any transmissions from ESVs within the minimum distances shall be with the prior agreement of the concerned administration(s).

5	The potentially concerned administrations referred to in § 4 above, are those where fixed or mobile services are allocated in the Table of Frequency Allocations of the Radio Regulations:

| Frequency bands | Potentially concerned administrations |
| --- | --- |
| 5 925-6 425 MHz | All three Regions |
| 14-14.25 GHz | Countries listed in No. 5.505 |
| 14.25-14.3 GHz | Countries listed in Nos. 5.505, 5.508 and 5.509 |
| 14.3-14.4 GHz | Regions 1 and 3 |
| 14.4-14.5 GHz | All three Regions |

6	Administrations, in applying the minimum distance referred to in § 4 above, are encouraged to exclude those parts of their territory, such as remote small islands, where terrestrial services in the bands 5 925-6 425 MHz and 14-14.5 GHz are neither operating nor planned.

7	The ESV system shall include means of identification and mechanisms to terminate transmissions, on a mandatory basis, whenever the station operates outside its authorized geographic (see § 4 above) or operational limits.

8	Termination of transmissions as referred to in § 7 above shall be implemented in such a way that the corresponding mechanisms can not be bypassed on board the vessel, except under the provisions of No. 4.9.

9	ESVs shall be equipped so as to enable the licensing administration under the provisions of Article 18 to verify earth station performance and to terminate ESV transmissions immediately upon request by an administration whose services may be affected.

10	When ESVs operating beyond the territorial waters but within the minimum distance (as referred to in § 4 above) fail to comply with the terms required by the concerned administration pursuant to § 2 and 4, then that administration may:

–	request the ESV to comply with such terms or cease operation immediately; or

–	request the licensing administration to require such compliance or immediate cessation of the operation.

ANNEX  2  TO  draft  RESOLUTION  [ESV-APT]  (wrc-03)

Technical limitations applicable to ESVs transmitting in the bands
5 925-6 425 MHz and 14.0-14.5 GHz

|  | 5 925-6 425 MHz | 14.0-14.5 GHz |
| --- | --- | --- |
| Minimum diameter of ESV antenna | 2.4 m | 1.2 m |
| Tracking accuracy of ESV antenna | 0.2 peak | 0.2 peak |
| Maximum ESV e.i.r.p. spectral density toward the horizon | 17 dB(W/MHz) | 12.5 dB(W/MHz) |
| Total ESV e.i.r.p. per vessel towards the horizon | 20.8 dBW | 16.3 dBW |

Off-axis limits

For earth stations on board vessels operating in the 5 925-6 425 MHz band, at any angle φ specified below, off the main lobe axis of an earth station antenna, the maximum e.i.r.p. in any direction within 3° of the GSO shall not exceed the following values:

5 925-6 425 MHz

Angle off-axis		Maximum e.i.r.p. per 4 kHz band

2.5°	≤	φ	≤	7°	(32 – 25 log φ) dB(W/4 kHz)

7°	<	φ	≤	9.2°	11 dB(W/4 kHz)

9.2°	<	φ	≤	48°	(35 – 25 log φ) dB(W/4 kHz)

48°	<	φ	<	180°	–7 dB(W/4 kHz)

For earth stations on board vessels operating in the 14.0-14.5 GHz band, at any angle φ specified below, off the main lobe axis of an earth-station antenna, the maximum e.i.r.p. in any direction within 3° of the GSO shall not exceed the following values:

14.0-14.5 GHz

Angle off-axis			Maximum e.i.r.p. in any 40 kHz band

2°	≤	φ	≤	7°		33 – 25 log φ dBW

7°	<	φ	≤	9.2	°	12 dBW

9.2°	<	φ	≤	48°		36 – 25 log φ dBW

φ	>	48°		–6 dBW

Coordination agreements between fixed-satellite service networks under Article 9 may result in lower off-axis e.i.r.p. levels.

Reasons:	The APT supports to provide protection to existing radio services, and to provide administrations operating systems in existing radio services with provision on how to reach agreement with operators of ESV systems as well as to provide administrations with the means to operate ESVs in the bands identified. The parameters in Annex 2 are in line with the ITU-R Study Group 4 Recommendation on ESV characteristics. The limits to the off-axis e.i.r.p. performance of ESVs operating in FSS networks are consistent with that of earth stations already operating in these networks in these bands, and to ensure efficient use of the GSO.

The APT generally supports the example Resolution [ESV 4.4-1] in Annex 4.4.2 of the CPM Report, except for the limitation of “Maximum occupied bandwidth per vessel”. Based on the studies in draft Recommendation ITU-R SF.[ESV-A] (Doc. 4/95–9/154), the APT believes that the limitation of “Maximum occupied bandwidth per vessel” will unduly constrain the ESVs operation.

SUP	ASP/25/180

RESOLUTION  82  (WRC-2000)

Provisions relating to earth stations located on board vessels 
which operate in fixed-satellite service networks in the 
bands 3 700-4 200 MHz and 5 925-6 425 MHz

Reasons:	Resolution 82 (WRC-2000) has been replaced by Resolution [ESV-APT] (WRC-03).

_______________

The reference point may be based on any method as per the relevant Articles of the United Nations Convention on the Law of the Sea of 10 December 1982, as required by the coastal State(s).

2 	The reference point may be based on any method as per the relevant Articles of the United Nations Convention on the Law of the Sea of 10 December 1982, as required by the coastal State(s).