MOD	ASP/25/181

d)	with respect to its conformity with the appropriate Regional Plan or the Regions 1 and 3 List, however, having characteristics differing from those in the appropriate Regional Plan or in the Regions 1 and 3 List, in one or more of the following aspects:

–	use of a reduced e.i.r.p.,

–	use of a reduced coverage area entirely situated within the coverage area appearing in the appropriate Regional Plan or in the Regions 1 and 3 List,

–	use of other modulating signals in accordance with the provisions of § 3.1.3 of Annex 5,

–	use of the assignment for transmission in the fixed-satellite service in accordance with No. 5.492,

–	in the case of Region 2, use of an orbital position under the conditions specified in § B of Annex 7,

–	in the case of the notification of Plan assignments, use of an e.i.r.p. which produces a pfd that exceeds the pfd limit of –103.6 dB(W/(m2 · 27 MHz)) given in Section 1 of Annex 1 to Appendix 30 on the territory of the notifying administration under the condition that the pfd on all the test points of those notified assignments are equal to or below those of the associated Plan assignments; or

...

Reasons:	It is needed to clarify that when administrations are implementing satellite networks under Article 5, the pfd limit may be exceeded on the territory of the notifying administration, as long as the pfd of the original Plan assignments at each test point of that assignment is not exceeded.