MOD	ASP/25/184

# 3	Limits to the change in the power flux-density to protect the broadcasting-satellite service in Regions 1 and 2 in the band 12.2-12.5 GHz and in Region 3 in the band 12.5-12.7 GHz

With respect to § 4.1.1 c) of Article 4, an administration in Region 2 is considered as being affected if the proposed new or modified assignment in the Regions 1 and 3 List would result in exceeding the power flux-densities given below, at any test point in the service area of its overlapping frequency assignments.

With respect to § 4.2.3 a), 4.2.3 b) or 4.2.3 f) of Article 4, as appropriate, an administration in Region 1 or 3 is considered as being affected if the proposed modification to the Region 2 Plan would result in exceeding the power flux-densities given below, at any test point in the service area of its overlapping frequency assignments:

–147	dB(W/(m2 · 27 MHz))	for    0°	 θ < 0.23°

–135.7 + 17.74 log θ	dB(W/(m2 · 27 MHz))	for    0.23°	 θ < 2.0°

–136.7 + 1.66 θ2	dB(W/(m2 · 27 MHz))	for    2.0°	 θ < 3.59°

–129.2 + 25 log θ	dB(W/(m2 · 27 MHz))	for    3.59°	 θ < 10.57°

–103.6	dB(W/(m2 · 27 MHz))	for    10.57°	 θ

where θ is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies.

(Proposal number ASP/25/185 not used.)