MOD	ASP/25/186

# 6	Limits to the change in the power flux-density of assignments in the Regions 1 and 3 Plan or List to protect the fixed-satellite service (space-to-Earth) in the band 11.7-12.2 GHz in Region 2 or in the band 12.2-12.5 GHz in Region 3, and of assignments in the Region 2 Plan to protect the fixed-satellite service (space-to-Earth) in the band 12.5-12.7 GHz in Region 1 and in the band 12.2-12.7 GHz in Region 3

With respect to § 4.1.1 e) of Article 4, an administration is considered as being affected if the proposed new or modified assignment in the Regions 1 and 3 List would result in an increase in the power flux-density over any portion of the service area of its overlapping frequency assignments in the fixed-satellite service in Region 2 or Region 3 of 0.25 dB or more above that resulting from the frequency assignments in the Plan or List for Regions 1 and 3 as established by WRC-2000.

With respect to § 4.2.3 e), an administration is considered as being affected if the proposed modification to the Region 2 Plan would result in an increase in the power flux-density over any portion of the service area of its overlapping frequency assignments in the fixed-satellite service in Region 1 or 3 of 0.25 dB or more above that resulting from the frequency assignments in the Region 2 Plan at the time of entry into force of the Final Acts of the 1985 Conference.

With respect to § 4.1.1 e) or 4.2.3 e) of Article 4, an administration is considered as not being affected if the proposed new or modified assignment in the Regions 1 and 3 List, or if a proposed modification to the Region 2 Plan, gives a power flux-density anywhere over any portion of the service area of its overlapping frequency assignments in the fixed-satellite service in Region 1, 2 or 3 of less than:

–186.5	dB(W/(m2 · 40 kHz)) 	for    0°	 θ < 0.054°

–164.0 + 17.74 log θ	dB(W/(m2 · 40 kHz))	for     0.054°	 θ < 2.0°

–165.0 + 1.66 θ2	dB(W/(m2 · 40 kHz))	for     2.0°	 θ < 3.59°

–157.5 + 25 log θ	dB(W/(m2 · 40 kHz))	for     3.59°	 θ < 10.57°

–131.9	dB(W/(m2 · 40 kHz))	for     10.57°	 θ

where θ is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies.

In the case an administration in Region 3 intends to protect its FSS assignments, which have been notified to the Bureau, brought into use and the relevant information as described in the Annex to Resolution 49 (Rev.WRC-2000) has been received by the Bureau before [DD.MM.YYYY], the administration is considered as not being affected if the proposed new or modified assignment in the Region 1 List, or if a proposed modification to the Region 2 Plan, gives a power flux-density anywhere over any portion of its own territory of its overlapping frequency assignments in its FSS networks less than:

–186.5	dB(W/(m2 · 40 kHz)) 	for     0°	 θ < 0.054°

–164.0 + 17.74 log θ	dB(W/(m2 · 40 kHz))	for     0.054°	 θ < 1.8°

–162.3 + 0.89 θ2	dB(W/(m2 · 40 kHz))	for     1.8°	 θ < 5.0°

–157.5 + 25 log θ	dB(W/(m2 · 40 kHz))	for     5.0°	 θ < 10.57°

–131.9	dB(W/(m2 · 40 kHz))	for   1 0.57°	 θ

where θ is the minimum geocentric orbital separation between the wanted and interfering space stations taking into account the respective East-West station-keeping accuracies.

-