| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 3 to Document 25-E |
|  |  |  | 15 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Asia-Pacific Common Proposals |  |
| proposals for the work of the conference |  |
| agenda item 1.3 |  |

1.3	to consider identification of globally/regionally harmonized bands, to the extent practicable, for the implementation of future advanced solutions to meet the needs of public protection agencies, including those dealing with emergency situations and disaster relief, and to make regulatory provisions, as necessary, taking into account Resolution 645 (WRC-2000)

Introduction

Identification of globally harmonized spectrum for PPDR is timely and important and is an urgent task in support of the global war on terrorism. This will help the PPDR agencies to work together through interoperability and enabling economies of scale necessary to lower the costs of specialized telecommunications systems to publicly funded entities tasked with protecting human life and property. APT strongly supports the identification of globally/regionally harmonized bands for PPDR at WRC-03 as it brings significant long-term benefits to administrations and their PPDR user communities including improved spectral efficiency and reducing the need for periodic band replanning. APT does not view public protection as separate and distinct from disaster relief for purposes of this agenda item. This agenda item is also most urgent because natural disasters such as earthquakes, fires and floods; and man-made disasters occur continually, unexpectedly and catastrophically in all nations. APT prefers Method C of the CPM Report and proposes identification of global or regional bands for PPDR through footnotes in the ITU Radio Regulations Article 5, with an appropriate reference to a Resolution.

Proposals

ARTICLE  5

ADD	ASP/25/28

5.PPDR	The bands 406.1-430 MHz, 440-470 MHz, 806-824 MHz, 851-869 MHz, 4 900-4 990 MHz and 5 850-5 925 MHz may be used to meet the needs of public protection and disaster relief applications by administrations wishing to use them for such applications. Such use does not preclude the use of these bands by any application in services to which these bands are allocated and does not establish priority in the Radio Regulations. Such use also does not preclude the use of

any other frequencies for public protection and disaster relief applications in accordance with the Radio Regulations. Resolution [PPDR] (WRC-03) provides administrations with guidance on use of these bands for public protection and disaster relief applications.

Reasons:	It is proposed to identify the globally/regionally harmonized bands for implementation of future advanced solutions by way of a footnote in the Table of Frequency Allocations, since this method gives the most visibility in the Radio Regulations to provide guidance to equipment manufacturers and to administrations, consistent with the objective of this agenda item. A supporting Resolution is proposed to give further context and guidance. Recognizing the need for flexibility to accommodate different national spectrum allocation priorities, the overall regulatory construction is explicitly designed such that neither the implementation of PPDR applications nor the clearance of existing services is mandatory. Administrations will retain the choice on which bands or parts thereof they use for PPDR applications or whether bands identified in this footnote (5.PPDR) are used for other services permitted by the Regulations.

MOD	ASP/25/29

335.4-410 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 406.1-410 FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY 5.149 ADD 5.PPDR |  |  |

MOD	ASP/25/29A

410-470 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 410-420 FIXED MOBILE except aeronautical mobile SPACE RESEARCH (space-to-space) 5.268 ADD 5.PPDR |  |  |
| 420-430 FIXED MOBILE except aeronautical mobile Radiolocation 5.269 5.270 5.271 ADD 5.PPDR |  |  |
| ... |  |  |
| 440-450 FIXED MOBILE except aeronautical mobile Radiolocation 5.269 5.270 5.271 5.284 5.285 5.286 ADD 5.PPDR |  |  |

| 450-455 FIXED MOBILE 5.209 5.271 5.286 5.286A 5.286B 5.286C 5.286D 5.286E ADD 5.PPDR |  |  |
| --- | --- | --- |
| 455-456 FIXED MOBILE | 455-456 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C | 455-456 FIXED MOBILE |
| 5.209 5.271 5.286A 5.286B 5.286C 5.286E ADD 5.PPDR | 5.209 ADD 5.PPDR | 5.209 5.271 5.286A 5.286B 5.286C 5.286E ADD 5.PPDR |
| 456-459 FIXED MOBILE 5.271 5.287 5.288 ADD 5.PPDR |  |  |
| 459-460 FIXED MOBILE | 459-460 FIXED MOBILE MOBILE-SATELLITE (Earth-to-space) 5.286A 5.286B 5.286C | 459-460 FIXED MOBILE |
| 5.209 5.271 5.286A 5.286B 5.286C 5.286E ADD 5.PPDR | 5.209 ADD 5.PPDR | 5.209 5.271 5.286A 5.286B 5.286C 5.286E ADD 5.PPDR |
| 460-470 FIXED MOBILE Meteorological-Satellite (space-to-Earth) 5.287 5.288 5.289 5.290 ADD 5.PPDR |  |  |

MOD	ASP/25/29B

470-890 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 470-790 BROADCASTING 5.149 5.291A 5.294 5.296 5.300 5.302 5.304 5.306 5.311 5.312 790-862 FIXED BROADCASTING 5.312 5.314 5.315 5.316 5.319 5.321 ADD 5.PPDR | 470-512 BROADCASTING Fixed Mobile 5.292 5.293 512-608 BROADCASTING 5.297 608-614 RADIO ASTRONOMY Mobile-satellite except aeronautical mobile-satellite (Earth-to-space) 614-806 BROADCASTING Fixed Mobile 5.293 5.309 5.311 806-890 FIXED MOBILE 5.317A BROADCASTING | 470-585 FIXED MOBILE BROADCASTING 5.291 5.298 585-610 FIXED MOBILE BROADCASTING RADIONAVIGATION 5.149 5.305 5.306 5.307 610-890 FIXED MOBILE 5.317A BROADCASTING |
| 862-890 FIXED MOBILE except aeronautical mobile 5.317A BROADCASTING 5.322 |  |  |
| 5.319 5.323 ADD 5.PPDR | 5.317 5.318 ADD 5.PPDR | 5.149 5.305 5.306 5.307 5.311 5.320 ADD 5.PPDR |

MOD	ASP/25/29C

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 4 800-4 990 FIXED MOBILE 5.442 Radio astronomy 5.149 5.339 5.443 ADD 5.PPDR |  |  |

MOD	ASP/25/29D

5 830-7 550 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Amateur Radiolocation | 5 850-5 925 FIXED FIXED-SATELLITE (Earth-to-space) MOBILE Radiolocation |
| 5.150 ADD 5.PPDR | 5.150 ADD 5.PPDR | 5.150 ADD 5.PPDR |

ADD	ASP/25/30

DRAFT  RESOLUTION  [PPDR]  (WRC-03)

Public protection and disaster relief

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	the growing telecommunication needs of public agencies and organizations dealing with law and order, disaster relief and emergency response;

b)	that future advanced solutions used by such public protection and disaster relief agencies and organizations will require high data rates;

c)	that there is a need for interoperability and interworking between public protection and disaster relief (PPDR) networks, both nationally and for cross-border operations, in emergency situations and disaster relief;

d)	the importance of the needs of public protection agencies and organizations, including those dealing with emergency situations and disaster relief for:

–	maintenance of law and order;

–	emergency and disaster response;

–	protection of life and property;

e)	that the provision of appropriate spectrum resources to PPDR is becoming increasingly important to the maintenance of a stable and prosperous society;

f)	that current PPDR applications are mostly narrow-band, including voice and low data-rate applications, typically in channel bandwidths of 25 kHz or less;

g)	that although there will continue to be narrow-band requirements, many future applications will be wideband (indicative data rates in the range of 384-500 kbit/s) and/or broadband (indicative data rates in the range of 1-100 Mbit/s) with channel bandwidths dependent on the use of spectrally efficient technologies;

h)	that new technologies for wideband and broadband PPDR applications are being developed in various standards organizations: in particular, a joint standardization programme

(known as Mobility for Emergency and Safety Applications (MESA)) between ETSI and TIA has commenced for broadband PPDR;

i)	that there is potential for new technologies such as IMT-2000 and systems beyond IMT-2000 and intelligent transportation systems (ITS) which may support or supplement advanced PPDR applications;

j)	that commercial systems may serve as a complement to dedicated systems in support of PPDR and that such complementary use would be in response to market demands;

k)	that Resolution 98 (Minneapolis, 1998) of the Plenipotentiary Conference urges Member States to facilitate use of telecommunications for the safety and security of humanitarian personnel;

l)	the special needs of developing countries,

recognizing

a)	the importance of interoperability in the provision of spectrum for public protection and disaster relief;

b)	the benefits of globally and regionally harmonized frequency bands, such as:

–	increased potential for interoperability;

–	a broader manufacturing base and increased volume of equipment resulting in economies of scale and expanded availability of equipment;

–	improved spectrum management and planning;

–	enhanced cross-border coordination;

–	improved cross-border circulation of equipment;

c)	that spectrum planning for PPDR is done at the national level, taking into account the need for interoperability and benefits of harmonization with neighbouring administrations;

d)	the increased benefits of cooperation between countries for the provision of effective and appropriate humanitarian assistance during disasters;

e)	the special needs of developing countries, taking into account the ITU-D Handbook on disaster relief;

f)	the needs of countries, particularly developing countries, for low-cost communications equipment for public protection and disaster relief agencies and organizations;

g)	that spectrum and innovative spectrum management may be required for the implementation of future advanced solutions;

h)	that the trend is to increase the use of technologies based on Internet Protocols;

i)	that currently some bands or parts thereof below 1 GHz have been designated for exclusive use for public protection and disaster relief; specifically:

–	some administrations in Region 2 have designated the bands 821-824/866-869 MHz for public protection and disaster relief use;

–	one administration in Region 2 has designated the bands 764-776/794-806 MHz for wideband PPDR;

–	one administration in Region 2 has designated the band 4 940-4 990 MHz for broadband PPDR;

–	harmonization, to some extent, has been achieved by some administrations in Region 1 by designating the frequency bands 380-385/390-395 MHz for emergency services;

–	some administrations in Region 3 are using, plan to use or have identified parts of the frequency bands 68-88 MHz, 138-144 MHz, 148-174 MHz, 380-399.9 MHz, 406.1-430 MHz, 440-502 MHz, 746-806 MHz, 806-824 MHz and 851-869 MHz for PPDR applications,

noting

a)	that many administrations use frequency bands below 1 GHz for narrow-band PPDR applications;

b)	applications requiring large coverage areas and providing good signal availability would generally be accommodated in lower frequency bands;

c)	applications requiring wider bandwidths would generally be accommodated in progressively higher bands;

d)	that PPDR agencies and organizations have a minimum set of requirements, including but not limited to, interoperability with national network and multiple agencies, group communications, secure and reliable communications, sufficient capacity to respond to emergencies, priority access in use of non-dedicated systems, fast response times, ability to handle multiple group calls and ability to cover large areas;

e)	that in most administrations, PPDR applications are provided at several levels, from national down to local, and cooperation between the levels is a domestic matter, which harmonized spectrum and interoperability could facilitate;

f)	that, while harmonization may be one method of realizing the benefits stated above, in some countries multiple frequency bands can be a component of meeting the communication needs in disaster situations;

g)	that WRC-03 has identified the frequency bands 406.1-430 MHz. 440-470 MHz, 806-824 MHz, 851-869 MHz, 4 900-4 990 MHz and 5 850-5 925 MHz for public protection and disaster relief applications in the mobile service under No. 5.PPDR of the Table of Frequency Allocations in the Radio Regulations;

h)	that, in addition to the frequency bands identified in No. 5.PPDR, other bands such as parts of 148-174 MHz, 380-399.9 MHz and 746-806 MHz are being extensively used for public protection and disaster relief applications. The band 380-385/390-395 MHz has been identified by some administrations for emergency communications and may continue to be used for providing public protection and disaster relief applications,

resolves

1	to urge administrations to use globally and regionally harmonized bands mentioned in noting g) and h) for public protection and disaster relief applications to the maximum extent possible;

2	to provide flexibility for disaster relief agencies and organizations by indicating that the identification of bands specifically for PPDR does not preclude the use of these bands by any application in services to which these bands are allocated and does not preclude the use of any other frequencies for disaster relief applications in accordance with the Radio Regulations;

3	to urge administrations, for advance planning, in emergency and disaster relief situations, to satisfy temporary needs for frequencies in addition to what may be normally provided for in agreements with neighbouring administrations;

4	to urge administrations to encourage PPDR agencies and organizations to utilize both existing and new technologies and solutions (satellite and terrestrial), to the extent practicable, to satisfy PPDR interoperability requirements and to further the goals of public protection and disaster relief;

5	that administrations should encourage agencies and organizations to use existing and advanced wireless solutions for providing complementary support to PPDR agencies and organizations,

resolves to recommend

1	that as necessary, administrations continue to work closely with their public protection and disaster relief community to further refine the identification of future spectrum requirements and possible methods to meet these requirements;

2	that administrations encourage PPDR agencies and organizations to utilize relevant ITU-R Recommendations in planning and implementing spectrum, technology and systems for public protection and disaster relief;

3	that equipment be made capable of operating in any part of an identified band, to provide administrations with spectrum management flexibility,

invites ITU-R

to conduct appropriate technical studies in support of the implementation of PPDR applications in the identified bands.

_______________

Some administrations believe that IMT-2000 is also an example of such advanced wireless solutions.