MOD	ASP/25/207

11.31	a)	with respect to its conformity with the Table of Frequency AllocationsMOD 8 and the other provisions9 of these Regulations, except those relating to conformity with the procedures for obtaining coordination or the probability of harmful interference, or those relating to conformity with a plan, as appropriate, which are the subject of the following sub-paragraphs;10

_______________

8	11.31.1	Conformity with the Table of Frequency Allocations implies the successful application of No. 9.21, when necessary. However, the recording of the assignment with respect to those administration(s) whose agreement(s) have not been obtained will be with favourable finding subject to the condition that the assignments in question shall not cause harmful interference to nor claim protection from the service(s) of the objecting administration(s) for which the agreement was sought.

Reasons:	To reflect in the Radio Regulations the concept of “partial conformity with No. 11.31” included in the Rules of Procedure in No. 11.31.1.