ADD	ASP/25/222

TABLE  21-4A

| Frequency band | Service* | Limit in dB(W/m2) for angle of arrival () above the horizontal plane |  |  |  | Reference bandwidth |
| --- | --- | --- | --- | --- | --- | --- |
| 1 520-1 525 MHz (Applicable to the territory of the United States in Region 2 between the longitudes 71° W and 125° W) | Mobile-satellite (space-to-Earth) | 0°  δ  4° | 4° < δ  20° | 20° δ 60° | 60°  δ  90° |  |
|  |  | –181.0 | –193.0  20 log δ | –213.3  35.6 log(δ) | –150.0 | 4 kHz |
| 1 520-1 525 MHz (Applicable to all other territory of the United States in Region 2) | Mobile-satellite (space-to-Earth | 0°  δ  43.4° |  | 43.4° < δ ≤ 60° | 60° < δ  90° |  |
|  |  | –155.0 |  | 213.3 + 35.6 log δ | –150.0 | 4 kHz |

Reasons:	The limit is to protect the AMT stations in the mobile service, based on CPM text (see § 2.8.1.4).