MOD	ASP/25/231

TABLE  7a

Parameters required for the determination of coordination distance for a transmitting earth station

| Transmitting space radiocommunication service designation |  | ... | Mobile- satellite |  | ... |
| --- | --- | --- | --- | --- | --- |
| Frequency bands (MHz) |  |  | 1 670-1 675 |  |  |
| Receiving terrestrial service designations |  |  | Fixed, mobile |  |  |
| Method to be used |  |  | § 1.4.6 |  |  |
| Modulation at terrestrial station 1 |  |  | A | N |  |
| Terrestrial station interference parameters and criteria | p0 (%) |  | 0.01 | 0.01 |  |
|  | n |  | 2 | 2 |  |
|  | p (%) |  | 0.005 | 0.005 |  |
|  | NL (dB) |  | 0 | 0 |  |
|  | Ms (dB) |  | 33 | 33 |  |
|  | W (dB) |  | 0 | 0 |  |
| Terrestrial station parameters | Gx (dBi) 3 |  | 35 | 35 |  |
|  | Te (K) |  | 750 | 750 |  |
| Reference bandwidth | B (Hz) |  | 4  103 | 106 |  |
| Permissible interference power | Pr( p) (dBW) in B |  | –131 | –107 |  |
| 1 A: analogue modulation; N: digital modulation. 2 The parameters for the terrestrial station associated with transhorizon systems have been used. Line-of-sight radio-relay parameters associated with the frequency band 1 675-1 710 MHz may also be used to determine a supplementary contour. 3 Feeder losses are not included. |  |  |  |  |  |

Reasons:	The coordination distance with respect to MetAids is a predetermined coordination distance (see Appendix 7, Table 10), hence no entry is required in Table 7. Further consequential modifications are made to the frequency ranges.