MOD	ASP/25/232

TABLE  9a

Parameters required for the determination of coordination distance for a transmitting earth station
in bands shared bidirectionally with receiving earth stations

| Space service designation in which the transmitting earth station operates |  | ... | Mobile-satellite |  | ... |
| --- | --- | --- | --- | --- | --- |
| Frequency bands (GHz) |  |  | 1.670-1.675 |  |  |
| Space service designation in which the receiving earth station operates |  |  | Meteorological-satellite |  |  |
| Orbit 6 |  |  | Non-GSO | GSO |  |
| Modulation at receiving earth station 1 |  |  | N | N |  |
| Receiving earth station interference parameters and criteria | p0 (%) |  | 0.006 | 0.011 |  |
|  | n |  | 3 | 2 |  |
|  | p (%) |  | 0.002 | 0.0055 |  |
|  | NL (dB) |  | 0 | 0 |  |
|  | Ms (dB) |  | 2.8 | 0.9 |  |
|  | W (dB) |  | 0 | 0 |  |
| Receiving earth station parameters | Gm (dBi) 2 |  | 30 | 45 |  |
|  | Gr (dBi) 4 |  | 19 9 | 8 |  |
|  | min 5 |  | 5 | 3 |  |
|  | Te (K) 7 |  | 370 | 118 |  |
| Reference bandwidth | B (Hz) |  | 106 | 4  103 |  |
| Permissible interference power | Pr( p) (dBW) in B |  | –145 | –178 |  |

Reasons:	Consequential modifications to the frequency ranges in the Tables of Appendix 7.