| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 37 to Document 25-E |
|  |  |  | 15 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Asia-Pacific Common Proposals |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| agenda item 1.37 |  |

1.37	to consider the regulatory and technical provisions for satellite networks using highly elliptical orbits

Introduction

620-790 MHz band

APT proposes that, in providing the guidance solicited by BR on how to deal with HEO filings in this band, WRC-03 adopt Resolution [UHF-TV] and modify No. 5.311 of the 2001 edition, as shown in Annex 1.37 below, with the effect of:

•	Grandfathering existing filings based on interim limits, subject to a review of findings by the Director of the Radiocommunication Bureau based on results of future sharing studies.

•	Suspending examination of new filings (post-WRC-03) until sharing studies called for produce more accurate sharing criteria, including adequate pfd limits, which should be considered by WRC-07.

•	Incorporating an interim normalized (1 MHz bandwidth) pfd mask within the Resolution [UHF-TV] to correct the ambiguity in the current No. 5.311 pfd limit and the current ambiguous and incomplete mask in Recommendation 705.

•	Modifying No. 5.311 of the 2001 edition, so as to make possible implementation of the new Resolution [UHF-TV], and remove the pfd limit in the existing version of No. 5.311, as it is now incorporated into the pfd mask in the new Resolution [UHF-TV].

Arising from these actions, APT has also made a proposal, under WRC-03 agenda item 7.2, so that the ITU-R studies called for by the new Resolution [UHF-TV] can be reviewed for appropriate action by WRC-07. Once these studies are completed, Recommendation 705 could be suppressed.

Proposals

ADD	ASP/25/257

RESOLUTION  [ASP/1.37/UHF-TV]  (WRC-03)

Technical and regulatory provisions relating to broadcasting-satellite 
networks operating in the 620-790 MHz band

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that No. 5.311 of the 2001 edition of the Radio Regulations provides the conditions under which the band 620-790 MHz, allocated to the broadcasting service on a primary basis, and to the fixed and mobile services on a primary basis in Region 3, may be used for assignments using frequency modulated broadcasting-satellite services;

b)	that it is necessary to have a power flux-density (pfd) limit which will provide adequate protection to the terrestrial television broadcasting service;

c)	that the regulatory and technical provisions for satellite networks using non-geostationary orbits, including highly elliptical orbits, are under study in ITU-R with respect to the impact of such systems on the terrestrial television broadcasting services;

d)	that GSO BSS and non-GSO BSS satellite systems are at the stage of advance publication and/or coordination, or have been notified in frequency bands from below 620 MHz to 790 MHz;

e)	that there are at present few GSO systems operating in accordance with No. 5.311 of the 2001 edition of the Radio Regulations;

f)	that it would be inappropriate to draw any conclusions regarding the form and levels of the protection criteria and their modes of application to GSO BSS and non-GSO BSS systems until the relevant studies are completed and the results are contained in ITU-R Recommendations;

g)	that many administrations have extensive infrastructure for the transmission and reception of analogue and digital television services between 620 MHz to 790 MHz, some services using ERPs of up to 5 MW, with significant potential for interference to BSS receive terminals with simple antennas,

noting

a)	that the CPM-02 Report concludes that the protection of terrestrial television services in the band 620-790 MHz requires more study before any conclusion can be made about the appropriate pfd values;

b)	the detailed information in former CCIR Report 631 concerning the incomplete studies forming the foundation of No. 5.311 of the 2001 edition of the Radio Regulations and Recommendation 705;

c)	that the studies leading to the development of No. 5.311 of the 2001 edition of the Radio Regulations were based on a limited number of GSO BSS television systems using analogue frequency modulation and analogue terrestrial television, with no account taken of digitally modulated GSO and non-GSO BSS systems and digital terrestrial television systems;

d)	that the reference bandwidth of the pfd limit in No. 5.311 of the 2001 edition of the Radio Regulations is undefined and guidance is urgently needed and requested by the Radiocommunication Bureau;

e)	that the interim studies in noting b) above did not account for multiple-entry aggregate interference from GSO and non-GSO BSS systems;

f)	that the urgent studies referred to in Recommendation 705 were never completed and it is therefore inappropriate to apply No. 5.311 of the 2001 edition of the Radio Regulations in the context of non-GSO BSS systems, or as planning criteria for RRC-04/05;

g)	that this Conference has suspended the application of Recommendation 705 and modified No. 5.311 of the Radio Regulations,

resolves

1	that the application of provisions of Articles 9 and 11 to GSO BSS and non-GSO BSS satellite networks, in the band 620-790 MHz for which the API has been received by BR after [4 July 2003] shall be suspended until ITU-R develops appropriate technical and regulatory guidance, based on new studies taking account of considering a) to g) above;

2	that, before an administration notifies to the Radiocommunication Bureau or brings into use a frequency assignment in the broadcasting-satellite service in the band 620-790 MHz, and for which the API has been received before [4 July 2003], it shall effect coordination of this assignment in accordance with No. 9.11 with any administration having a primary allocation to terrestrial services in the same frequency band on whose territory the pfd exceeds the following thresholds:

–142.8 		dB(W/(m2 · MHz))			for					    20

–142.8 + 0.4 ( – 20)	dB(W/(m2 · MHz))		for		20	<      60

–126.8 		dB(W/(m2 · MHz))				for		60	<      90

where  is the angle of arrival above the horizontal plane (degrees);

3	the limits in resolves 2 are hard limits for   20, which may be exceeded on the territory of any country whose administration has so agreed,

invites ITU-R

to conduct studies as a matter of urgency, and develop sharing criteria and regulatory provisions in time for WRC-07, for protection of terrestrial primary services, in particular terrestrial television broadcasting services, in the 620-790 MHz band from GSO BSS and non-GSO BSS satellite services operating or planning to operate in this band,

instructs the Director of the Radiocommunication Bureau

to review, and revise if necessary, the findings previously made under resolves 2 and 3 based on the results of sharing studies done by ITU-R and decisions of WRC-07 with respect to these studies.

Reasons:	To address current ambiguities in the application of Recommendation 705 and to replace Recommendation 705 applicability effected through No. 5.311 by prescribed transitional arrangements in Resolution [UHF-TV], while awaiting the results of further studies which are called for in the Resolution. When studies are completed in time for WRC-07, Recommendation 705 could be suppressed at WRC-07.

ARTICLE  5

MOD	ASP/25/258

5.311	Within the frequency band 620-790 MHz, assignments may be made to television stations using frequency modulation in the broadcasting-satellite service subject to the provisions of Resolution [UHF-TV] (WRC-03) and the agreement to be made between the administrations concerned and those having services, operating in accordance with the Table, which may be affected (see Articles 9 and 11 and Resolution 507, except resolves 2).-

Reasons:	To identify the new Resolution that captures the appropriate transitional arrangements to be followed and remove mask characteristics, which are no longer appropriate.

APPENDIX  5  (WRC-2000)

MOD	ASP/25/259

TABLE  5-1 (continued)

| Reference of Article 9 | Case | Frequency bands (and Region) of the service for which coordination is sought | Threshold/condition | Calculation method | Remarks |
| --- | --- | --- | --- | --- | --- |
| No. 9.11 GSO, non-GSO/ terrestrial | A space station in the broadcasting-satellite service (BSS) in any band shared on an equal primary basis with terrestrial services and where the BSS is not subject to a Plan, in respect of terrestrial services | 620-790 MHz 1 452-1 492 MHz 2 310-2 360 MHz 2 520-2 655 MHz 2 655-2 670 MHz 12.5-12.75 GHz (Region 3) 17.3-17.8 GHz (Region 2) 21.4-22 GHz (Region 1 and 3) 74-76 GHz | Bandwidths overlap; Resolution 539 (WRC-2000); Resolution [UHF-TV] (WRC-03) applies in the 620-790 MHz band | Check by using the assigned frequencies and bandwidths |  |
| . . . |  |  |  |  |  |

Reasons:	To include a reference to Resolution [UHF-TV] as a consequence of adopting this Resolution.

__________