ASP/25/264

The Asia-Pacific Telecommunity proposes the following updates to references to ITU-R Recommendations in the 2001 version of the Radio Regulations. These concern Recommendations that have been revised since WRC-2000.

| Action | Provision No. | Provision | Reason |
| --- | --- | --- | --- |
| (MOD) | 5.288 | In the territorial waters of the United States and the Philippines, the preferred frequencies for use by on-board communication stations shall be 457.525 MHz, 457.550 MHz, 457.575 MHz and 457.600 MHz paired, respectively, with 467.750 MHz, 467.775 MHz, 467.800 MHz and 467.825 MHz. The characteristics of the equipment used shall conform to those specified in Recommendation ITU-R M.1174-1 (Geneva, 1998). | Language is already clearly mandatory and the MOD only aligns the text with the Recommendation as listed in Volume 4 of the Radio Regulations. |
| (MOD) | TABLE 22-1A (WRC-2000) Reference antenna column | Limits to the epfd radiated by non-geostationary-satellite systems in the fixed-satellite service systems in certain frequency bands3, 4, 5, 6 Recommendation ITU-R S.1428-1 (Geneva, 2001) | MOD to indicate latest revision of the Recommendation. Replace the reference in Volume 4 of the Radio Regulations with the latest revision. |
| - (MOD) | TABLE 22-1B (WRC-2000) Reference antenna column | Limits to the epfd¯ radiated by non-geostationary-satellite systems in the fixed-satellite service in certain frequency bands3, 6, 8 Recommendation ITU-R S.1428-1 (Geneva, 2001) | MOD to indicate latest revision of the Recommendation. Replace the reference in Volume 4 of the Radio Regulations with the latest revision. |
| (MOD) | TABLE 22-1C (WRC-2000) Reference antenna column | Limits to the epfd¯ radiated by non geostationary-satellite systems in the fixed-satellite service in certain frequency bands3, 6, 8 Recommendation ITU-R S.1428-1 (Geneva, 2001) | MOD to indicate latest revision of the Recommendation. Replace the reference in Volume 4 of the Radio Regulations with the latest revision. |
| - (MOD) | 7 22.5C.6 | For this Table, reference patterns of Recommendation ITU-R S.1428-1 (Geneva, 2001) shall be used only for the calculation of interference from non-geostationary-satellite systems in the fixed-satellite service systems into geostationary-satellite systems in the fixed-satellite service. (WRC-2000) | MOD consequential to changes to Tables 22-1A, 22-1B and 22-1C above to add revision number to Recommendation. |

| Action | Provision No. | Provision | Reason |
| --- | --- | --- | --- |
| - (MOD) | TABLE 22-1D (WRC-2000) Reference antenna column | Limits to the epfd¯ radiated by non-geostationary-satellite systems in the fixed-satellite service in certain frequency bands into 30 cm, 45 cm, 60 cm, 90 cm, 120 cm, 180 cm, 240 cm and 300 cm broadcasting-satellite service antennas6, 9, 10, 11 Recommendation ITU-BO.1443-1 (Geneva, 2002) Annex 1 | MOD to indicate latest revision of the Recommendation. Replace the reference in Volume 4 of the Radio Regulations with the latest revision. |
| - SUP | Incorporated Recommendation ITU-R S.1428 |  | SUP as the Recommendation has been superseded by a later version. |
| - ADD | Incorporated Recommendation ITU-R S.1428-1 | [Add text of Recommendation to Volume 4] | ADD latest version of Recommendation as a consequence of changing Tables 22-1A, 22-1B and 22-1C. |
| - SUP | Incorporated Recommendation ITU-BO.1443 |  | SUP as the Recommendation has been superseded by a later version. |
| - ADD | Incorporated Recommendation ITU-BO.1443-1 | [Add text of Recommendation to Volume 4] | ADD latest version of Recommendation as a consequence of changing Table 22-1D. |

Amendment of references in RR Volume 1 to ITU-R Recommendations 
related to specific WRC-03 agenda items

ARTICLE  52

Special rules relating to the use of frequencies

Section II  –  Use of frequencies for Morse radiotelegraphy

B1  –  Call and reply