MOD	ASP/25/283

RESOLUTION  33  (Rev.WRC-03)

Bringing into use of space stations in the broadcasting-satellite service,
prior to the entry into force of agreements and associated plans for
the broadcasting-satellite service

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that while Resolution 507 envisages plans for the broadcasting-satellite service (BSS), some administrations might nevertheless feel the need to bring stations in that service into use prior to such plans being established;

b)	that administrations should, as far as possible, avoid proliferation of space stations in the BSS before such plans have been established;

c)	that a space station in the BSS may cause harmful interference to terrestrial stations operating in the same frequency band, even if the latter are outside the service area of the space station;

d)	that the procedures specified in Articles 9 to 14 and Appendix 5 contain provisions for coordination between stations in the BSS and terrestrial stations, between space systems in that service and space systems of other administrations;

e)	that there are many existing and planned stations in the BSS not subject to agreements and associated plans that have submitted advance publication information (API) or a request for coordination under the existing Resolution 33 procedures and that some administrations are currently in coordination under these procedures,

resolves

1	that, except in those cases where agreements and associated plans for the BSS have been established and have entered into force, for satellite networks for which the API has been received following 1 January 1999, only the procedures of Articles 9 to 14 shall be applied for the coordination and notification of stations in the BSS and coordination and notification of other services in respect of that service;

2	that, except in those cases where agreements and associated plans for the BSS have been established and have entered into force, for satellite networks for which the API has been received by the Radiocommunication Bureau prior to 1 January 1999, only the procedure in Sections A to C in this Resolution shall be applied;

3	that a future conference review the requirement for the procedures in this Resolution.

Section A  –  Coordination procedure between space stations in the broadcasting-satellite service and terrestrial stations

2.1	Before an administration notifies to the Bureau or brings into use any frequency assignment to a space station in the broadcasting-satellite service in a frequency band where this frequency band is allocated, with equal rights, to the broadcasting-satellite service and to a terrestrial radiocommunication service, either in the same Region or sub-Region or in different Regions or sub-Regions, it shall coordinate the use of this assignment with any other administration whose terrestrial radiocommunication services may be affected. For this purpose, it shall inform the Bureau of all the technical characteristics of the station, as listed in the relevant sections of Appendix 4 to the Radio Regulations, which are necessary to assess the risk of interference to a terrestrial radiocommunication service1.

2.2	The Bureau shall publish this information in a Special section of its Weekly Circular and shall also, when the Weekly Circular contains such information, so advise all administrations by circular telegram.

2.3	Any administration which considers that its terrestrial radiocommunication services may be affected shall forward its comments to the administration seeking coordination and, in any case, to the Bureau. These comments must be forwarded within four months from the date of the relevant BR IFIC. It shall be deemed that any administration which has not forwarded comments within that period considers that its terrestrial radiocommunication services are unlikely to be affected.

2.4	Any administration which has forwarded comments on the projected station shall either give its agreement, with a copy to the Bureau, or, if this is not possible, send to the administration seeking coordination all the data on which its comments are based as well as any suggestions it may be able to offer with a view to a satisfactory solution of the problem.

2.5	The administration which plans to bring into use a space station in the broadcasting-satellite service as well as any other administration which believes that its terrestrial radiocommunication services are likely to be affected by the station in question may request the assistance of the Bureau at any time during the coordination procedure.

2.6	In the event of continuing disagreement between an administration seeking to effect coordination and one with which coordination has been sought, the administration seeking coordination shall, except in the cases where the assistance of the Bureau has been requested, defer the submission of its notice concerning the proposed assignment by six months from the date of publication of the information according to § 2.2.

Section B  –  Coordination procedure between space stations in the broadcasting-satellite service and space systems of other administrations

3	An administration intending to bring into use a space station in the broadcasting-satellite service shall, for the purpose of coordination with space systems of other administrations, apply the following provisions of Article 11 of the Radio Regulations (edition of 1990, revised in 1994):

3.1	Nos. 1041 to 1058 inclusive.

3.2.1	Nos. 1060 to 10652.

3.2.2	No coordination under § 3.2.1 is required when an administration proposes to change the characteristics of an existing assignment in such a way as not to increase the probability of harmful interference to stations in the space radiocommunication service of other administrations.

3.2.3	Nos. 1074 to 1105 inclusive.

Section C  –  Notification, examination and recording in the Master Register of
assignments to space stations in the broadcasting-satellite service dealt
with under this resolution

4.1	Any frequency assignment3 to a space station in the broadcasting-satellite service shall be notified to the Bureau. The notifying administration shall apply for this purpose the provisions of Nos. 1495 to 1497 of the Radio Regulations (edition of 1990, revised in 1994).

4.2	Notices made under § 4.1 shall initially be treated in accordance with No. 1498 of the Radio Regulations (edition of 1990, revised in 1994).

5.1	The Bureau shall examine each notice with respect to:

5.2	a)	its conformity with the Convention, the Table of Frequency Allocations and the other provisions of the Radio Regulations, with the exception of those relating to the coordination procedures and to the probability of harmful interference, which are the subject of § 5.3, 5.4, and 5.5;

5.3	b)	its conformity, where applicable, with the provisions of § 2.1 of Section A above, relating to coordination of the use of the frequency assignment with the other administrations concerned;

5.4	c)	its conformity, where applicable, with the provisions of § 3.2.1 of Section B above, relating to coordination of the use of the frequency assignment with the other administrations concerned;

5.5	d)	where appropriate, the probability of harmful interference to the service rendered by a station in a space or terrestrial radiocommunication service for which a frequency assignment has already been recorded in the Master Register in conformity with the provisions of No. 1240 or 1503 of the Radio Regulations (edition of 1990, revised in 1994), or No. 11.31, as appropriate, if that assignment has not, in fact, caused harmful interference to the service rendered by a station for which an assignment has been previously recorded in the Master Register and which itself is in conformity with No. 1240 or 1503 of the Radio Regulations (edition of 1990, revised in 1994), or No. 11.31, as appropriate.

6.1	Depending upon the findings of the Bureau subsequent to the examination prescribed in § 5.2, 5.3, 5.4 and 5.5, further action shall be as follows:

6.2	Where the Bureau reaches an unfavourable finding with respect to § 5.2, the notice shall be returned immediately by airmail to the notifying administration with the reasons of the Bureau for this finding together with such suggestions as the Bureau is able to offer with a view to a satisfactory solution of the problem.

6.3	Where the Bureau reaches a favourable finding with respect to § 5.2, or where it reaches the same finding after resubmission of the notice, it shall examine the notice with respect to the provisions of § 5.3 and 5.4.

6.4	Where the Bureau finds that the coordination procedures mentioned in § 5.3 and 5.4 have been successfully completed with all administrations whose services may be affected, the assignment shall be recorded in the Master Register. The date of receipt by the Bureau of the notice shall be entered in Column 2d of the Master Register with an entry in the Remarks Column indicating that such recording does not prejudge in any way the decisions to be included in the agreements and associated plans referred to in Resolution 507.

6.5	Where the Bureau finds that the coordination procedures mentioned in § 5.3 or 5.4 have not, as appropriate, been applied or have been unsuccessfully applied, the notice shall be returned immediately by airmail to the notifying administration with the reason for its return together with such suggestions as the Bureau is able to offer with a view to a satisfactory solution of the problem.

6.6	Where the notifying administration resubmits the notice and states that it has been unsuccessful in endeavouring to effect the coordination, the notice shall be examined by the Bureau with respect to § 5.5.

6.7	Where the notifying administration resubmits the notice and the Bureau finds that the coordination procedures have been successfully completed with all administrations whose services may be affected, the assignment shall be treated as indicated in § 6.4.

6.8	Where the Bureau reaches a favourable finding with respect to § 5.5, the assignment shall be recorded in the Master Register. The appropriate symbol indicating the finding by the Bureau shall indicate that the coordination procedures, as appropriate, referred to in § 2.1 or 3.2.1 were not successfully completed. The date of receipt by the Bureau of the notice shall be entered in Column 2d of the Master Register, with the remark mentioned in § 6.4.

6.9	Where the Bureau reaches an unfavourable finding with respect to § 5.5, the notice shall be returned immediately by airmail to the notifying administration with the reasons for the Bureau’s finding together with such suggestions as the Bureau is able to offer with a view to a satisfactory solution of the problem.

6.10	If the administration resubmits the notice unchanged with the insistence that it be reconsidered, but should the Bureau’s unfavourable finding under § 5.5 remain unchanged, the assignment shall be recorded in the Master Register. However, this entry shall be made only if the notifying administration informs the Bureau that the assignment has been in use for at least four months without any complaint of harmful interference having been received. The date of receipt by the Bureau of the original notice shall be entered in Column 2d of the Master Register, with the remark mentioned in § 6.4. An appropriate remark shall be placed in Column 13 to indicate that the assignment is not in conformity with the provisions of § 5.3, 5.4 or 5.5, as appropriate. In the event that the administration concerned receives no complaint of harmful interference concerning the operation of the station in question for a period of one year from the commencement of operation, the Bureau shall review its finding.

6.11	If harmful interference is actually caused to the reception of any space station in the broadcasting-satellite service whose frequency assignment has been recorded in the Master Register as a result of a favourable finding with respect to § 5.2, 5.3, 5.4 and 5.5 of this Resolution, as appropriate, by the use of a frequency assignment to a space station which has been subsequently recorded in the Master Register in accordance with the provisions of § 6.10 of this Resolution or of No. 1544 of the Radio Regulations (edition of 1990, revised in 1994), or No. 11.41, as appropriate, the station using the latter frequency assignment must, upon receipt of advice thereof, immediately eliminate this harmful interference.

6.12	If harmful interference is actually caused to the reception of any space radiocommunication station using an assignment recorded in the Master Register as a result of a favourable finding with respect to Nos. 1503 to 1512 of the Radio Regulations (edition of 1990, revised in 1994), or Nos. 11.31 to 11.34, as appropriate, by the use of an assignment to a space station in the broadcasting-satellite service which has been subsequently recorded in the Master Register in accordance with the provisions of § 6.10 of this Resolution, the station using the latter assignment must, on receipt of advice thereof, immediately eliminate this harmful interference.

6.13	If harmful interference is actually caused to the reception of any terrestrial station using an assignment recorded in the Master Register as a result of a favourable finding with respect to No. 1240 of the Radio Regulations (edition of 1990, revised in 1994), or No. 11.31, as appropriate, by the use of an assignment to a space station in the broadcasting-satellite service which has been subsequently recorded in the Master Register in accordance with the provisions of § 6.10 of this Resolution, the station, using the latter assignment must, on receipt of advice thereof, immediately eliminate this harmful interference.

6.14	If harmful interference to the reception of any station whose assignment is in accordance with § 5.2 of this Resolution is actually caused by the use of a frequency assignment which is not in conformity with No. 1240, 1352 or 1503 of the Radio Regulations (edition of 1990, revised in 1994), or No. 11.31, as appropriate, the station using the latter frequency assignment must, upon receipt of advice thereof, immediately eliminate this harmful interference.

Reasons:	Modifications to the text of Resolution 33, in order clarify the text and to remove the ambiguities currently existing in the text of the Resolution.

1 	The calculation methods and the interference criteria to be employed in evaluating the interference should be based upon relevant ITU-R Recommendations agreed by the administrations concerned either as a result of Resolution 703 (Rev.WARC-92) or otherwise. In the event of disagreement on an ITU-R Recommendation or in the absence of such Recommendations, the methods and criteria shall be agreed between the administrations concerned. Such agreements shall be concluded without prejudice to other administrations.

2 	See footnote 1.

3 	The expression frequency assignment, wherever it appears in this Resolution, shall be understood to refer either to a new frequency assignment or to a change in an assignment already recorded in the Master International Frequency Register (hereinafter called the Master Register).