ADD	ASP/25/288

draft  RESOLUTION  [ASP/7.2/XXX]  (WRC-03)

Consideration of regulatory provisions for global protection 
of passive sensors operating in the band 10.6-10.68 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the frequency band 10.6-10.68 GHz is allocated to the Earth exploration-satellite service (EESS) (passive) on a primary basis;

b)	that frequency band 10.6-10.68 GHz is also allocated to the fixed and mobile (except aeronautical mobile) services on a primary basis and a large number of such systems are in operation in accordance with No. 5.482 of the Radio Regulations;

c)	that No. 5.482 of the Radio Regulation stipulates that stations of the fixed and mobile, except aeronautical mobile, services shall be limited to a maximum equivalent isotropically radiated power of 40 dBW and the power delivered to the antenna shall not exceed –3 dBW;

d)	that the frequency band 10.6-10.68 GHz is important for measurement of global water circulation and, in particular, indispensable for estimating precipitation, sea surface wind speed, sea surface temperature and soil moisture, in combination with other bands;

e)	that when the fixed and mobile (except aeronautical mobile) services are allowed to operate without the constraint which are shown in No. 5.482, enough protection may not be given to the EESS (passive),

resolves

1	that with regard to No. 5.482, WRC-07 should verify its efficiency and effectiveness for its global application;

2	that WRC-07 should also consider the result of ITU-R studies to review the regulatory provisions for global protection of passive sensors,

requests the ITU-R

1	to develop the technical and operational characteristics of the passive spaceborne sensor systems in the band 10.6-10.68 GHz;

2	to verify the criteria currently contained in No. 5.482 of the Radio Regulations by which passive spaceborne sensor systems can share with the fixed and mobile (except aeronautical mobile) services in this band, with a view to applying its efficiency and effectiveness of the criteria globally.