ADD	ASP/25/290

draft  RESOLUTION  [ASP/7.2/ZZZ]  (WRC-03)

Use of the frequency band 1 215-1 260 MHz and 1 260-1 300 MHz
by spaceborne active sensors

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the bands 1 215-1 260 MHz and 1 260-1 300 MHz are allocated to the Earth exploration-satellite service (EESS) (active) on a primary basis with the constraint of No. 5.332 and No. 5.335A;

b)	that the frequency bands 1 215-1 260 MHz and 1 260-1 300 MHz are also allocated to the radiolocation and radionavigation satellite (space-to-Earth) (space-to-space) services on a primary basis;

c)	that the frequency bands 1 215-1 260 MHz and 1 260-1 300 MHz are used for spaceborne active sensors for Earth observation;

d)	that spaceborne active sensors are important for cloud-free and day-and-night Earth observation, and useful for cartography, environment monitoring, disaster monitoring and resource surveying;

e)	that Nos. 5.332 and 5.335A stipulate that spaceborne active sensors for Earth observation shall not cause harmful interference to, nor claim protection from, the other services allocated on a primary basis;

f)	that this restriction makes the EESS almost a secondary allocation in practice;

g)	that studies have shown that sharing between spaceborne active sensors and radiolocation systems in the bands 1 215-1 260 MHz and 1 260-1 300 MHz is feasible;

h)	that it is possible to enhance the compatibility between spaceborne active sensors and radiolocation systems by introducing the mitigation techniques, which are shown in the PDNR of the Chairman’s Report of Working Party 7C;

i)	that Recommendation ITU-R SA.1347 indicates that sharing is feasible between radionavigation-satellite systems and spaceborne synthetic aperture radars in the 1 215-1 260 MHz frequency band,

resolves

1	that ITU-R is invited to continue to study sharing between spaceborne active sensors for Earth observation and other services in the bands 1 215-1 260 MHz and 1 260-1 300 MHz;

2	to recommend that WRC-07 review the results of ITU-R studies with a view to examining the necessity of the restrictions currently imposed in the Nos. 5.332 and 5.335A on spaceborne active sensors in the EESS operating in the bands 1 215-1 260 MHz and 1 260-1 300 MHz without any retroactive impact on the submissions received by the Radiocommunication Bureau before [end WRC-2003].