| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 5 to Document 25-E |
|  |  |  | 15 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Asia-Pacific Common Proposals |  |
| proposals for the work of the conference |  |
| Agenda item 1.5 |  |

1.5	to consider, in accordance with Resolution 736 (WRC-2000), regulatory provisions and spectrum requirements for new and additional allocations to the mobile, fixed, Earth exploration-satellite and space research services, and to review the status of the radiolocation service in the frequency range 5 150-5 725 MHz, with a view to upgrading it, taking into account the results of ITU-R studies

Introduction

Resolution 736 (WRC-2000) includes four resolves. APT noted that allocations or upgrade under resolves 1, 3 and 4 are closely related and is of the view that allocations and upgrade under these three resolves should be considered together. Also allocation of the mobile service in the band 5 150-5 250 MHz needs to be considered in conjunction with agenda item 1.6.

APT basically supports Method A presented in Section 2.2.3.1.1 of the CPM Report for resolves 1, Method A in Section 2.2.3.2.1 for resolves 2, Method A in Section 2.2.3.3.1 for resolves 3 and Method A in Section 2.2.3.4.1 for resolves 4.

Proposals

article  5

MOD	ASP/25/34

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 150-5 250 AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE ADD 5.WAS1 5.446 5.447B 5.447C |  |  |

MOD	ASP/25/34A

| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) MOBILE ADD 5.WAS2 RADIOLOCATION SPACE RESEARCH 5.447D | 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) FIXED ADD 5.FWA1 MOBILE ADD 5.WAS2 RADIOLOCATION SPACE RESEARCH 5.447D |
| --- | --- |
| 5.448 | 5.448 |
| 5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) MOBILE ADD 5.WAS2 RADIOLOCATION SPACE RESEARCH (active) | 5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) FIXED ADD 5.FWA1 MOBILE ADD 5.WAS2 RADIOLOCATION SPACE RESEARCH (active) |
| 5.448 | 5.448 |

MOD	ASP/25/34B

5 350-5 460 EARTH EXPLORATION-SATELLITE (active) 5.448B AERONAUTICAL RADIONAVIGATION 5.449 RADIOLOCATION

MOD	ASP/25/34C

5 460-5 470 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B RADIONAVIGATION 5.449 SPACE RESEARCH (active) ADD 5.SRS RADIOLOCATION

MOD	ASP/25/34D

5 470-5 570 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B SPACE RESEARCH (active) ADD 5.SRS MARITIME RADIONAVIGATION MOBILE ADD 5.WAS3 RADIOLOCATION 5.450 5.451 5.452

MOD	ASP/25/34E

5 570-5 650 MARITIME RADIONAVIGATION MOBILE ADD 5.WAS3 RADIOLOCATION 5.450 5.451 5.452

MOD	ASP/25/34F

5 650-5 725 MOBILE ADD 5.WAS3 RADIOLOCATION Amateur Space research (deep space) 5.282 5.451 5.453 5.454 5.455

Reasons:	5 150-5 350 MHz: APT supports the new primary allocation to the mobile service in the bands 5 150-5 350 MHz for the implementation of WAS (wireless access systems) including RLANs.

In order to ensure protection of the existing services, new footnotes specifying the operational and technical constraints for these new applications should be added (see ASP/25/35 and 37). APT also supports the new primary allocation to the fixed service in Region 3 in the bands 5 250-5 350 MHz (see ASP/25/38).

Reasons:	5 350-5 725 MHz: APT supports the new primary allocation to the mobile service in the bands 5 470-5 725 MHz for the implementation of WAS (wireless access systems) including RLANs. In order to ensure protection of the existing services, a new footnote specifying the operational and technical constraints for this application should be added (see ASP/25/39).

APT also supports the primary allocations for the Earth exploration-satellite (active) and space research (active) services in the frequency band 5 460-5 570 MHz, provided that this band is also allocated to the mobile service on a primary basis.

The constraints stated in the current footnote 5.448B for EESS (active) should be extended accordingly to the band 5 460-5 570 MHz (see ASP/25/41), and the same kind of constraints should be specified also to the space research (active) service (see ASP/25/42).

The radiolocation service in the band 5 350-5 650 MHz needs to be upgraded when the band is allocated to the mobile service on a primary basis, in order to provide the radiolocation service with an equal status to the mobile service.

ADD	ASP/25/35

5.WAS1	Use of the 5 150-5 250 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems (WAS), including RLANs, which shall be operated in accordance with the following conditions (see also Resolution [RLAN 2.3-1] (WRC-03)):

–	deployment of WAS, including RLANs, shall be restricted to indoor use only;

–	the mean equivalent isotropically radiated power density limit of any WAS station, including RLAN stations, shall not exceed –20 dBW in any 1 MHz band (or equivalently –44 dBW in any 4 kHz band).

In the band 5 150-5 250 MHz stations in the mobile service shall not claim protection from earth stations in the fixed-satellite service. The provisions of No. 5.43A do not apply to the mobile service allocation in the band 5 150-5 250 MHz.

NOTE – ASP/25/35 and ASP/25/45 are identical.

Reasons:	In order to protect feeder links (Earth-to-space) for non-GSO MSS, the above technical and operational restrictions should be applied to WAS including RLANs.

SUP	ASP/25/36

5.447

NOTE – ASP/25/36 and ASP/25/46 are identical.

Reasons: 	Suppression of No. 5.447 is conditional to the primary allocation of the band 5 150-5 250 MHz to the mobile service as proposed in ASP/25/34.

ADD	ASP/25/37

5.WAS2	Use of the 5 250-5 350 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems (WAS), including RLANs, which shall be operated in accordance with the following conditions:

–	deployment of WAS, including RLANs, shall be restricted to indoor use only with a mean equivalent isotropically radiated power (e.i.r.p.) limit of –7 dBW and a mean e.i.r.p. density limit of –20 dBW in any 1 MHz band;

–	WAS stations, including RLAN stations, shall implement transmitter power control (TPC) to ensure a mitigation factor of at least 3 dB on the average output power. If TPC is not implemented, the e.i.r.p. limits given above shall be reduced by 3 dB;

–	WAS stations, including RLAN stations, shall implement dynamic frequency selection (DFS) techniques described in [draft new] Recommendation ITU-R M.[8A-9B.RLAN.DFS] to avoid co-channel operation with other terrestrial systems, notably radar systems. The DFS devices shall also be designed to ensure a near-uniform spread of the loading of the devices across the available spectrum to improve the sharing with satellite services.

Reasons:	In order to protect the existing services in the band 5 250-5 350 MHz the above technical and operational conditions should be applied to WAS, including RLANs.

ADD	ASP/25/38

5.FWA1	The use of the frequency band 5 250-5 350 MHz by the fixed service is permitted in Region 3 for fixed wireless access systems subject to compliance with Recommendation ITU-R F.[Doc. 9/BL/46, FWA5GHz-EESS]. In addition, FWA systems shall not claim protection from the radiodetermination, Earth exploration-satellite (active) and space research (active) services, although the Earth exploration-satellite (active) and space research (active) services are not subject to No. 5.43A. After coordination of FWA systems to protect the existing radiodetermination systems, future radiodetermination systems shall not impose more stringent coordination constraints on the FWA systems.

Reasons:	To enable operation of FWA systems in Region 3 countries while protecting the existing services in the band 5 250-5 350 MHz.

ADD	ASP/25/39

5.WAS3	Use of the 5 470-5 725 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems (WAS), including RLANs, which shall be operated in accordance with the conditions below:

–	the indoor and outdoor use of WAS, including RLANs, shall be restricted to a mean e.i.r.p. limit of 0 dBW and a mean e.i.r.p. density limit of –13 dBW in any 1 MHz;

–	WAS stations, including RLAN stations, shall implement transmitter power control (TPC) to ensure a mitigation factor of at least 3 dB on the average output power. If TPC is not implemented, the e.i.r.p. limits given above shall be reduced by 3 dB;

–	WAS stations, including RLAN stations, shall implement dynamic frequency selection (DFS) techniques described in [draft new] Recommendation ITU-R M.[8A-9B.RLAN.DFS] to avoid co-channel operation with other terrestrial systems, notably radar systems. The DFS devices shall also be designed to ensure a near uniform spread of the loading of the devices across the available spectrum to improve the sharing with satellite services.

Reasons:	The above technical and operational conditions should be applied to WAS including RLANs in order to protect the existing services or other services to which new allocation is considered in the 5 470-5 725 MHz range.

SUP	ASP/25/40

5.448A

Reasons:	As operational experience shows the successful sharing between the EESS (active) and the RLS, the removal of No. 5.448A would not constrain the development and deployment of the radiolocation service.

MOD	ASP/25/41

5.448B	The Earth exploration-satellite (active) service operating in the band 5 350-5 570 MHz shall not cause harmful interference to, or constrain the use and development of, the radionavigation service.   (WRC-03)

Reasons:	Consequential to extension of allocation for the Earth exploration-satellite (active) service to the band 5 460-5 570 MHz.

ADD	ASP/25/42

5.SRS	The space research (active) service operating in the band 5 460-5 570 MHz shall not cause harmful interference to, or constrain the use and development of, the radionavigation service.

Reasons:	For protection of the radionavigation service, the same constraints as those on the Earth exploration-satellite (active) service should be specified also to the space research (active) service.

SUP	ASP/25/43

RESOLUTION  736  (WRC-2000)

Consideration by a future competent world radiocommunication conference
of issues dealing with allocations to the mobile, fixed, radiolocation,
Earth exploration-satellite (active), and space research (active)
services in the frequency range 5 150-5 725 MHz

Reasons:	All the resolves in Resolution 736 (WRC-2000) will be satisfied in WRC-03.

____________