ADD	ASP/25/37

5.WAS2	Use of the 5 250-5 350 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems (WAS), including RLANs, which shall be operated in accordance with the following conditions:

–	deployment of WAS, including RLANs, shall be restricted to indoor use only with a mean equivalent isotropically radiated power (e.i.r.p.) limit of –7 dBW and a mean e.i.r.p. density limit of –20 dBW in any 1 MHz band;

–	WAS stations, including RLAN stations, shall implement transmitter power control (TPC) to ensure a mitigation factor of at least 3 dB on the average output power. If TPC is not implemented, the e.i.r.p. limits given above shall be reduced by 3 dB;

–	WAS stations, including RLAN stations, shall implement dynamic frequency selection (DFS) techniques described in [draft new] Recommendation ITU-R M.[8A-9B.RLAN.DFS] to avoid co-channel operation with other terrestrial systems, notably radar systems. The DFS devices shall also be designed to ensure a near-uniform spread of the loading of the devices across the available spectrum to improve the sharing with satellite services.

Reasons:	In order to protect the existing services in the band 5 250-5 350 MHz the above technical and operational conditions should be applied to WAS, including RLANs.