ADD	ASP/25/38

5.FWA1	The use of the frequency band 5 250-5 350 MHz by the fixed service is permitted in Region 3 for fixed wireless access systems subject to compliance with Recommendation ITU-R F.[Doc. 9/BL/46, FWA5GHz-EESS]. In addition, FWA systems shall not claim protection from the radiodetermination, Earth exploration-satellite (active) and space research (active) services, although the Earth exploration-satellite (active) and space research (active) services are not subject to No. 5.43A. After coordination of FWA systems to protect the existing radiodetermination systems, future radiodetermination systems shall not impose more stringent coordination constraints on the FWA systems.

Reasons:	To enable operation of FWA systems in Region 3 countries while protecting the existing services in the band 5 250-5 350 MHz.