ADD	ASP/25/39

5.WAS3	Use of the 5 470-5 725 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems (WAS), including RLANs, which shall be operated in accordance with the conditions below:

–	the indoor and outdoor use of WAS, including RLANs, shall be restricted to a mean e.i.r.p. limit of 0 dBW and a mean e.i.r.p. density limit of –13 dBW in any 1 MHz;

–	WAS stations, including RLAN stations, shall implement transmitter power control (TPC) to ensure a mitigation factor of at least 3 dB on the average output power. If TPC is not implemented, the e.i.r.p. limits given above shall be reduced by 3 dB;

–	WAS stations, including RLAN stations, shall implement dynamic frequency selection (DFS) techniques described in [draft new] Recommendation ITU-R M.[8A-9B.RLAN.DFS] to avoid co-channel operation with other terrestrial systems, notably radar systems. The DFS devices shall also be designed to ensure a near uniform spread of the loading of the devices across the available spectrum to improve the sharing with satellite services.

Reasons:	The above technical and operational conditions should be applied to WAS including RLANs in order to protect the existing services or other services to which new allocation is considered in the 5 470-5 725 MHz range.