ADD	ASP/25/45

5.WAS1	Use of the 5 150-5 250 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems (WAS) including RLANs, which shall be operated in accordance with the following conditions (see also Resolution [RLAN 2.3-1] (WRC-03)):

–	deployment of WAS, including RLANs, shall be restricted to indoor use only;

–	the mean equivalent isotropically radiated power density limit of any WAS station, including RLAN stations, shall not exceed –20 dBW in any 1 MHz band (or equivalently –44 dBW in any 4 kHz band).

In the band 5 150-5 250 MHz stations in the mobile service shall not claim protection from earth stations in the fixed-satellite service. The provisions of No. 5.43A do not apply to the mobile service allocation in the band 5 150-5 250 MHz.

NOTE – ASP/25/45 and ASP/25/35 are identical.

Reasons:	In order to protect feeder links (Earth-to-space) for non-GSO MSS, the above technical and operational restrictions should be applied to WAS including RLANs.