MOD	ASP/25/78

1 350-1 525 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 452-1 492 FIXED MOBILE except aeronautical mobile BROADCASTING 5.345 5.347 BROADCASTING- SATELLITE 5.345 5.347 ADD 5.AAA | 1 452-1 492 FIXED MOBILE 5.343 BROADCASTING 5.345 5.347 BROADCASTING-SATELLITE 5.345 5.347 ADD 5.AAA |  |
| 5.341 5.342 | 5.341 5.344 |  |

Reasons:	The footnote No. 5.340 applies to the band 1 400-1 427 MHz. Thus “All emissions are prohibited” in the band 1 400-1 427 MHz. ITU-R studies have resulted that, in the band 1 400-1 427 MHz allocated to the RAS on a primary basis, the unwanted emissions produced by space stations in the BSS (space-to-Earth) operating in the 1 452-1 492 MHz can meet the protection criteria for the VLBI and single dish (continuum and spectral line) observations. However, to meet the spectral line protection criteria, it is likely that mitigation techniques such as filtering would have to be implemented. Thus it is possible to establish mandatory pfd limits, contained in the proposed new footnote No. 5.AAA (ASP/25/79), to the unwanted emissions produced by space stations in the BSS (space-to-Earth) to protect specific types of the radio astronomy service in the band 1 400-1 427 MHz, without unduly constraining operations of space stations in the BSS in the band 1 452-1 492 MHz.