ADD	ASP/25/81

5.BBB	The power flux-density produced by any geostationary space station in the mobile-satellite service (space-to-Earth) operating in the band 1 525-1 559 MHz shall not exceed at a radio astronomy station (depending on the type of observations conducted):

–	180 dB(W/m2) in 27 MHz (for single dish continuum observations) in the band 1 400-1 427 MHz;

–	–196 dB(W/m2) in any 20 kHz (for single dish spectral-line observations) in the band 1 400-1 427 MHz; or

–	–166 dB(W/m2) in any 20 kHz (for VLBI observations) in the band 1 400-1 427 MHz.

These values shall apply to the space station for which advance publication information (API) has been received after the end of WRC-03, with regard to any radio astronomy station that has been notified to ITU prior to the receipt of the API of the space station. Other radio astronomy stations, notified afterwards, may seek an agreement of administrations authorizing the space station.

Reasons:	The footnote No. 5.340 applies to the band 1 400-1 427 MHz. Thus “All emissions are prohibited” in the band 1 400-1 427 MHz. ITU-R studies have resulted that in the band 1 400-1 427 MHz allocated to the RAS on a primary basis the unwanted emissions produced by space stations in the MSS (space-to-Earth) operating in the 1 525-1 559 MHz can meet the protection criteria for the VLBI and single dish (continuum and spectral line) observations. As is widely accepted in ITU-R, radio astronomy stations notified to BR earlier than the API of the space station need to be protected, and RAS stations notified later than the API of the space station need to seek an agreement if they hope to be protected.