MOD	ASP/25/84

1 610-1 660 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) ADD 5.CCC | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION RADIODETERMINATION- SATELLITE (Earth-to-space) Mobile-satellite (space-to-Earth) ADD 5.CCC | 1 613.8-1 626.5 MOBILE-SATELLITE (Earth-to-space) 5.351A AERONAUTICAL RADIONAVIGATION Mobile-satellite (space-to-Earth) ADD 5.CCC Radiodetermination-satellite (Earth-to-space) |
| 5.341 5.355 5.359 5.363 5.364 5.365 5.366 5.367 5.368 5.369 5.371 5.372 | 5.341 5.364 5.365 5.366 5.367 5.368 5.370 5.372 | 5.341 5.355 5.359 5.364 5.365 5.366 5.367 5.368 5.369 5.372 |

Reasons:	ITU-R studies have resulted in that, in the band 1 610.6-1 613.8 MHz allocated to the RAS on a primary basis, the unwanted emissions produced by space stations in the MSS (space-to-Earth) operating in the band 1 525-1 559 MHz, RNSS (space-to-Earth) operating in the band 1 559-1 610 MHz or MSS (space-to-Earth) operating in the band 1 613.8-1 626.5 MHz can meet the protection criteria for radio astronomical observations in the band 1 610.6-1 613.8 MHz for continuum, VLBI and spectral line observations.