MOD	ASP/25/85

10-11.7 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A ADD 5.DDD (Earth-to-space) 5.484 MOBILE except aeronautical mobile | 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A ADD 5.DDD MOBILE except aeronautical mobile | 10.7-11.7 FIXED FIXED-SATELLITE (space-to-Earth) 5.441 5.484A ADD 5.DDD MOBILE except aeronautical mobile |

Reasons:	ITU-R studies have resulted that, in the band 10.6-10.7 GHz allocated to the RAS on a primary basis, the unwanted emissions produced by space stations in the FSS (space-to-Earth) operating in the 10.7-10.95 GHz can meet the protection criteria for the VLBI observations only. Thus it is possible to establish mandatory pfd limits, contained in the proposed new footnote No. 5.DDD (ASP/25/86), to the unwanted emissions produced by space stations in the FSS (space-to-Earth) to protect specific types of the radio astronomy service in the band 10.6-10.7 GHz, without unduly constraining operations of space stations in the FSS in the band 10.7-10.95 GHz.