| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 9 to Document 25-E |
|  |  |  | 15 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Asia-Pacific Common Proposals |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| agenda item 1.9 |  |

1.9	to consider Appendix 13 and Resolution 331 (Rev.WRC-97) with a view to their deletion and, if appropriate, to consider related changes to Chapter SVII and other provisions of the Radio Regulations, as necessary, taking into account the continued transition to and introduction of the Global Maritime Distress and Safety System (GMDSS)

Introduction

APT Members note that Morse code proficiency, carriage of a radio officer, and SAR watchkeeping on 500 kHz are no longer a mandatory requirement internationally. However, some Members need to continue supporting large numbers of vessels that will take until around 2009 to convert to GMDSS. Therefore, APT Members propose as follows:

•	modify Chapter IX to delete the radiotelegraphy requirement from the radio officers’ certificate;

•	modify Resolution 331 (Rev.WRC-97);

•	make no change to Appendix 13.

Proposals

ARTICLE  47

Operator’s certificates

Section I  –  General provisions

SUP	ASP/25/93

47.1	§ 1	1)

Reasons:	All vessels subject to the SOLAS convention are regulated to comply with GMDSS from 1 February 1999, Morse code is no longer mandatory from that date and shore-based stations offering this service in most regions have ceased operating.

SUP	ASP/25/94

47.3		3)

Reasons:	Automatic Morse alarms are no longer carried on board vessels since the implementation of GMDSS and the closure of radiotelegraphy services.

Section IV  –  Qualifying service

MOD	ASP/25/95

47.26	§ 8	1)	The holder of a radiocommunication general operator’s certificate shall be authorized to embark as chief operator of a ship station of the fourth category (described in Recommendation ITU-R M.1169, Geneva, 1995).

Reasons:	Radiotelegraph qualifications are no longer required and to standardize the Recommendation reference in accordance with Resolution 27.

MOD	ASP/25/96

47.27		2)	However, before becoming chief or sole operator of a ship station of the fourth category (in accordance with Recommendation ITU-R M.1169, Geneva, 1995), the holder of a radiocommunication general operator’s certificate shall have had adequate experience as operator on board ship at sea.

Reasons:	Provisions on radiotelegraph operator certificates are no longer needed and to standardize Recommendation reference in accordance with Resolution 27.

MOD	ASP/25/97

47.28		3)	Before becoming chief operator of a ship station of the second or third category (described in Recommendation ITU-R M.1169, Geneva, 1995), the holder of a radiocommunication general operator’s certificate shall have had, as operator on board ship or in a coast station, at least six months’ experience of which at least three months shall have been on board ship.

Reasons:	Radiotelegraph qualifications are no longer required and to standardize Recommendation reference in accordance with Resolution 27.

MOD	ASP/25/98

47.29		4)	Before becoming chief operator of a ship station of the first category (in accordance with Recommendation ITU-R M.1169, Geneva, 1995), the holder of a radiocommunication general operator’s certificate shall have had, as operator on board ship or in a coast station, at least one year’s experience of which at least six months shall have been on board ship.

Reasons:	Radiotelegraph qualifications are no longer required and to standardize Recommendation reference in accordance with Resolution 27.

ARTICLE  55

Morse radiotelegraphy

MOD	ASP/25/99

55.1		The radiotelegraph procedure -may be conducted in accordance with the latest revision of Recommendation ITU-R M.1170.

Reasons:	To delete the reference to the use of Morse telegraphy for distress and safety communications whilst retaining the possibility of using Morse telegraphy for other purposes in accordance with Recommendation ITU-R M.1170.

NOC	ASP/25/100

APPENDIX  13  (WRC-2000)*

Distress and safety communications (non-GMDSS)

Reasons:	Some countries still use 500 kHz Morse telegraphy for distress communication.

MOD	ASP/25/101

RESOLUTION  331  (Rev.WRC-03)

Transition to the Global Maritime Distress and Safety System (GMDSS) and continuation of the distress and safety provisions in Appendix 13

The World Radiocommunication Conference (Geneva, 2003),

noting

that the International Convention for the Safety of Life at Sea (SOLAS), 1974, as amended, prescribes that all ships subject to this Convention shall be fitted for the Global Maritime Distress and Safety System (GMDSS) by 1 February 1999,

noting further

a)	that a number of administrations have taken steps to implement the GMDSS also for classes of vessels not subject to SOLAS, 1974, as amended;

b)	that an increasing number of vessels not subject to SOLAS, 1974, as amended, are making use of the techniques and frequencies of the GMDSS prescribed in Chapter VII;

c)	that some administrations and vessels, not subject to SOLAS, 1974, as amended, may wish to continue to use provisions of Appendix 13 for distress and safety communications for some time after 1 February 1999;

d)	that it would be costly for administrations to maintain in parallel for an excessive period of time shore-based facilities necessary to support both the old and new distress and safety systems;

e)	that there may be a need to continue existing shore-based distress and safety services for a certain period after 1 February 1999 so that vessels not subject to SOLAS, 1974, as amended and not yet using the techniques and frequencies of the GMDSS will be able to obtain assistance from these services until such time as they are able to participate in the GMDSS;

f)	that separate provisions of the existing Radio Regulations designate VHF channel 16 and the frequency 2 182 kHz as the international channels for general calling by radiotelephony;

g)	that the International Maritime Organization (IMO) has already decided for GMDSS vessels that:

–	listening watches on 2 182 kHz will no longer be mandatory after 1 February 1999;

–	listening watches on VHF channel 16 will be continued and that a final date for the cessation of mandatory watchkeeping on channel 16 by SOLAS ships while at sea will be reviewed prior to 2005;

h)	that the Radio Regulations require GMDSS ships to keep watch on the appropriate digital selective calling (DSC) distress frequencies;

i)	that the Radio Regulations establish that ship stations should, when practicable, keep watch on VHF channel 13;

j)	that several administrations have established Vessel Traffic Service (VTS) systems and require their ships to keep watch on local VTS channels;

k)	that ships that are required by SOLAS to carry a radio station are being equipped with DSC, but the majority of vessels that carry a radio station on a voluntary basis might not have DSC equipment;

l)	that similarly, many administrations have established distress and safety service based on DSC watchkeeping, but the majority of port stations, pilot stations and other operational coast stations have not been equipped with DSC facilities;

m)	that for the reasons listed above, it will remain necessary for some stations in the maritime mobile service to call each other by radiotelephony in certain situations,

considering

a)	that the operation of the GMDSS described in Chapter VII and the present distress and safety system described in Appendix 13 differ in many crucial aspects, such as means and methods of alerting, communication facilities available, announcement and transmission of maritime safety information, etc.;

b)	that operation of the two systems in parallel for a long period would cause ever-increasing difficulties and incompatibilities between ships operating in the two different systems and may thus seriously degrade safety at sea in general;

c)	that the GMDSS overcomes the deficiencies of the aural watch-keeping on maritime distress and calling frequencies on which the distress and safety system described in Appendix 13 relies, by replacing these watches by automatic watch, i.e. digital selective calling and satellite communication systems,

resolves

1	that, until such time as voice calling has become obsolete, VHF channel 16 and the frequency 2 182 kHz may be used as voice-calling channels;

2	to urge all administrations to assist in enhancing safety at sea by:

–	encouraging all ships to make use of the GMDSS as soon as possible;

–	encouraging, where appropriate, establishment of suitable shore-based facilities for GMDSS, either on an individual basis or in cooperation with other relevant parties in the area;

–	encouraging all ships carrying maritime VHF equipment to be fitted with DSC on VHF channel 70 as soon as possible, taking into account the relevant decisions of IMO;

3	that administrations may, taking account of all aspects involved, such as:

–	decisions by IMO on aural watch on 2 182 kHz and VHF channel 16;

–	the GMDSS radio systems available in the area concerned;

–	the compatibility problems mentioned in considering a) and b) above;

–	the density and classes of ships normally in the area;

–	the geographical nature of the area and general navigational conditions within the area;

–	other adequate measures taken to ensure safety communications for ships sailing in the area,

at a time after 1 February 1999, when the development on transition to the GMDSS and the prevailing conditions in the area makes it reasonable to do so, release their ship stations and coast stations within the area concerned from the obligations described in Appendix 13 on listening watch on 2 182 kHz or VHF channel 16 or both;

when doing so, administrations should:

–	inform IMO of their decisions and submit to IMO details on the area concerned;

–	inform the Secretary-General on the necessary details for inclusion in the List of Coast Stations,

resolves further

that the Secretary-General should ensure that such arrangements and details regarding the area concerned be indicated in relevant maritime publications,

invites the next world radiocommunication conference

to include the review of this Resolution, Appendix 13 and Chapter VII on the agenda of WRC-07,

instructs the Secretary-General

to communicate this Resolution to IMO and the International Civil Aviation Organization (ICAO),

invites the Radiocommunication Study Group 8

to review the operational and procedural incompatibilities between the old and new systems with a view to presenting the information to WRC-07.

Reasons:	Consideration should be given to retain Resolution 331 as guidance for administrations on the final transition to the GMDSS and to adjust the Resolution to reflect the developments since WRC-97.

__________