ADD	ARB/27A29/4

5.551GY	The power flux-density produced by any geostationary space station in the fixed-satellite service (space-to-Earth) or in the broadcasting-satellite service (space-to-Earth) operating in the band 42-42.5 GHz shall not exceed at a radio astronomy station (depending on the type of observations conducted):

–	137 dB(W/m2) in 1 GHz (for single dish continuum observations) in the band 42.5-43.5 GHz;

–	–153 dB(W/m2) in any 500 kHz (for single dish spectral-line observations) in the band 42.5*–43.5 GHz; or

–	–116 dB(W/m2) in any 500 kHz (for VLBI observations) in the band 42.5-43.5 GHz.

These values shall apply at any radio astronomy station that has been notified to ITU either before 4 July 2003 or before the date of receipt of the advance publication information (API) of the space station to which the limits are to apply. Other radio astronomy stations, notified after these dates, may seek an agreement with administrations authorizing the space stations.

Reasons:	Consequential to the Arab proposal and to the ITU-R studies.