ADD	ARB/27A39/4

The Arab Administrations support the identification of spectrum for systems beyond IMT-2000 subject to completion of ITU-R studies and also taking into account the needs of developing countries. The ITU-R studies on the identification of spectrum for systems beyond IMT-2000 should take into account the spectrum requirements of both the terrestrial and the satellite component of systems beyond IMT-2000.

# 3	Items for inclusion in the agendas of future conferences

### 3.1.1	resolves to give the view (3.2) of Resolution 801

to consider results of ITU-R studies in accordance with Resolution 222 (WRC-2000) to ensure spectrum availability and protection for the aeronautical mobile-satellite (R) service and the GMDSS, and to take appropriate action on this subject, while retaining the generic allocation for the MSS

### 3.1.2	Background/reason

Resolution 222 (WRC-2000) invites ITU-R to conduct studies on the spectrum availability and protection of AMS(R)S in the bands 1 545-1 555 MHz and 1 646.5-1 656.5 MHz, and GMDSS in the bands 1 530-1 544 MHz and 1 626.5-1 645.5 MHz.

WRC-97 allocated the frequency bands 1 525-1 559 MHz and 1 626.5-1 660.5 MHz on a generic basis to MSS. The spectrum requirements of MSS networks are currently coordinated through the capacity planning approach (multilateral coordination). This approach has proved its practicality and success for years. However, due to the general shortage of the spectrum available to the MSS, the approach might become unusable in the future.

Therefore Resolution 222 (WRC-2000) requests emphasis on studying the practicality and feasibility of inter-system prioritization and pre-emption.

### 3.1.3	Proposal

The Arab Administrations are of the following view: