MOD	ARB/27A4/3

RESOLUTION  114  (Rev.WRC-03)

Use of the band 5 091-5 150 MHz by the fixed-satellite service (Earth-to-space) (limited to feeder links of the non-geostationary mobile-satellite service)

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	the current allocation of the frequency band 5 000-5 250 MHz to the aeronautical radionavigation service;

b)	the requirements of both the aeronautical radionavigation and the fixed-satellite (Earth-to-space) (limited to feeder links of non-geostationary (non-GSO) mobile-satellite systems) services in the above-mentioned band,

recognizing

a)	that precedence must be given to the microwave landing system (MLS) in accordance with No. 5.444 of the Radio Regulations and to other international standard systems of the aeronautical radionavigation service in the frequency band 5 000-5 150 MHz;

b)	that, in accordance with Annex 10 of the Convention of the International Civil Aviation Organization (ICAO), it may be necessary to use the frequency band 5 091-5 150 MHz for the MLS if the requirements cannot be satisfied in the frequency band 5 030-5 091 MHz;

c)	that the fixed-satellite service providing feeder links for non-GSO mobile-satellite services will need access to the frequency band 5 091-5 150 MHz in the short term, in order to accommodate already identified requirements,

noting

a)	the necessary evolution of the current MLS and of other international standard systems in the aeronautical radionavigation service implementation plans;

b)	the small number of fixed-satellite service stations to be considered,

resolves

1	that administrations authorizing stations providing feeder links for non-GSO mobile-satellite systems in the frequency band 5 091-5 150 MHz shall ensure that they do not cause harmful interference to existing and future stations of the aeronautical radionavigation service;

2	that the allocation to the aeronautical radionavigation service and the fixed-satellite service in the frequency band 5 091-5 150 MHz should be reviewed at a future competent conference prior to 2018,

urges administrations

1	when authorizing stations of the aeronautical radionavigation service, to assign frequencies giving priority to the band below 5 091 MHz;

2	when assigning frequencies in the band 5 091-5 150 MHz before 1 January 2018 to stations of the aeronautical radionavigation service or to stations of the fixed-satellite service providing feeder links of the non-GSO mobile-satellite service (Earth-to-space), to take all practicable steps to avoid mutual interference between them,

instructs ITU-R

1	to study the technical and operational issues relating to sharing of this band between the aeronautical radionavigation service and the fixed-satellite service providing feeder links of the non-GSO mobile-satellite service (Earth-to-space);

2	to bring the results of these studies to the attention of a future competent conference prior to 2018,

invites

1	ICAO to further review, within the same time-frame, detailed spectrum requirements and planning for international standard aeronautical radionavigation systems in the above-mentioned band;

2	all members of the Radiocommunication Sector, and especially ICAO, to participate actively in such studies,

requests the Secretary-General

to bring this Resolution to the attention of ICAO.

Reasons:

1	The CPM text states, “The current radio regulatory mechanism, with an extension of the date 2010 to 2018, would meet the current aviation requirements”.

2	Due to the uncertainty as to whether RNSS can provide a future low visibility landing system, the expansion of microwave landing system (MLS) is not clear.

3	The time-frame when FSS becomes secondary to aeronautical radionavigation service will be postponed for an appropriate period because of uncertain future requirements and deployment of both ARNS and FSS.

4	Maintaining Resolution 114 will allow further studies between the two services in light of future developments expected.