| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 6 to Document 27-E |
|  |  |  | 16 April 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| Arab States Common Proposals |  |
| Proposals for the work of the conference |  |
| Agenda item 1.6 |  |

1.6	to consider regulatory measures to protect feeder links (Earth-to-space) for the mobile-satellite service which operate in the band 5 150-5 250 MHz, taking into account the latest ITU-R Recommendations (for example, Recommendations ITU-R S.1426, ITU-R S.1427 and ITU-R M.1454)

# 1	Introduction

The Arab Administrations support the additional allocation to the mobile service in the band 5 150-5 250 MHz. However, the Conference should take into consideration while adopting the new identification of mobile services in the 5 150-5 250 MHz the following regulatory provisions:

1)	A definition to WAS/RLAN shall be added and included in the proposed new Resolution.

2)	A Resolution inviting ITU-R to continue work on regulatory mechanisms and further mitigation techniques to ensure that the aggregate interference will not increase to a detrimental level from a possible prolific growth in the number of RLAN devices.

3)	The protection of the aeronautical radionavigation service in this band.

# 2	Proposals

Article 5 of the Radio Regulations should be amended to reflect the views expressed above as follows:

ARTICLE  5

MOD	ARB/27A6/1

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 150-5 250 AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE ADD 5.DDD 5.446 5.447 5.447B 5.447C |  |  |

ADD	ARB/27A6/2

5.DDD	In the band 5 150-5 250 MHz, the use of the mobile service shall be in accordance with Resolution [ARB-XXX-2] (WRC-03). Such use shall not cause harmful interference to, or claim protection from, aeronautical radionavigation or fixed-satellite services.

The adoption of No. 5.DDD shall not have any restrictions on the additional allocation stated in No. 5.447.

Reasons:

•	To allocate primary allocation to the mobile service for RLAN in line with the result of agenda item 1.5.

•	To protect existing services from harmful interferences.

•	To ensure that RLANs introduction will not limit the additional allocation in countries mentioned in No. 5.447.

ADD	ARB/27A6/3

Draft  RESOLUTION  [ARB-XXX-2]  (WRC-03)

Provisions to protect feeder links of non-geostationary-satellite systems 
in the mobile-satellite service in the 5 150-5 250 MHz band

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that the fixed-satellite service (Earth-to-space) is allocated worldwide on a primary basis in the band 5 150-5 250 MHz, this allocation being limited to feeder links of non-geostationary-satellite systems in the mobile-satellite service (MSS) (No. 5.447A);

b)	that the band 5 150-5 250 MHz is also allocated to the mobile service, on a primary basis, in some countries (No. 5.447, WRC-2000) subject to agreement obtained under No. 9.21, and that this allocation is already planned for use by RLANs in Europe on a co-primary basis;

c)	that RLAN devices are planned to be distributed on an unlicensed basis and to comply with the restrictions stated in resolves 1 below;

d)	that the interference from a single RLAN device complying with the operational restrictions in resolves 1 below will not on its own cause any unacceptable increase in the noise level at the satellite;

e)	that the MSS satellite receivers may experience an unacceptable effect due to the aggregate interference from these RLAN devices, especially in the case of a prolific growth in the number of these RLAN devices;

f)	that the aggregate effect will be due to the global deployment of RLAN devices and that it may not be possible to apportion the cause of the effect between individual administrations,

recognizing

a)	that a means is required to prevent the aggregate interference from the worldwide deployment of RLANs from becoming detrimental to the feeder links of non-geostationary-satellite systems in the MSS;

b)	that an aggregate pfd level has been developed in Recommendation ITU-R S.1426;

c)	that there is a degree of uncertainty in the means to measure or calculate the aggregate pfd level specified in Recommendation ITU-R S.1426,

resolves

1	that in the band 5 150-5 250 MHz, the use of WAS, including RLANs, shall be operated in accordance with the following conditions:

–	that administrations should ensure that the mean1 e.i.r.p. density limit of RLAN or other wireless access transmitter devices operating in the band 5 150-5 250 MHz should be no greater than 10 mW in any 1 MHz (or equivalently 0.04 mW in any 4 kHz) per transmitter (see Notes 1 and 2);

–	that, in addition, administrations should take measures to ensure that RLAN or other wireless access transmitters are operated indoors in the band 5 150-5 250 MHz;

2	to invite ITU-R to continue work on regulatory and technical procedures in order to address the aggregate interference from a possible prolific growth in the number of RLAN devices.

NOTE 1 – Annex 1 of Recommendation ITU-R M.1454 contains a methodology and parameters that have been used in sharing studies.

NOTE 2 – For RLAN carriers with less than 1 MHz bandwidth, the e.i.r.p. density should not exceed 0.01 W/Hz over the carrier bandwidth.

Reasons:	The Arab Administrations support the introduction of mobile services for wireless access systems including RLANs in the frequency band 5 150-5 250 MHz, taking into consideration the protection of the existing MSS feeder links in this band, operating under the fixed-satellite service (Earth-to-space) allocation. The Arab Administrations have noted that the following points have to be resolved by the ITU-R Study Groups:

•	Assessment of the aggregate interference at satellite receivers due to a number of low powered RLANs, due to the fact that non-GSO satellites can have large viewing areas therefore large capture areas for interference signals.

•	Assessment to the value when pfd limits of Recommendation ITU-R S.1426 are reached.

•	What actions should be taken when pfd limits/thresholds are reached.

Therefore, the Arab Administrations believe that the application of relevant operational limits from Recommendation ITU-R M.1454, combined with a new WRC-03 Resolution inviting further study, provides the most suitable solution placing necessary restrictions on RLANs, whilst providing a mechanism to continue technical and regulatory studies to deal with potential incompatibilities resulting from aggregate interference within an appropriate time-scale.

_____________

1 	The mean power refers here to the e.i.r.p. radiated during the transmission burst at the power control protocol which corresponds to the highest power, if power control is implemented.