MOD	IRN/32/44

5.502	In the band 13.75-14 GHz, an earth station of a geostationary fixed-satellite service network shall have a minimum antenna diameter of 1.2 m and an earth station of a non-geostationary fixed-satellite service system shall have a minimum antenna diameter of 4.5 m. In addition, the e.i.r.p. averaged over one second, radiated by a station in the radiolocation or radionavigation services shall not exceed 59 dBW above 2º elevations and 65 dBW below.- An administration planning to use FSS earth stations with an antenna diameter less than 4.5 m in a geostationary network in this band shall ensure that the single-entry power flux-density produced by any earth station operating within its territory does not exceed:

–	–105.2 dB(W/(m2  10 MHz)) not to be exceeded for more than 1.0% of the time produced at 36 m above sea level at the [normal baseline, straight baseline, baseline as defined in the UN Convention on the Law Of the Sea 1982 or at a point specified by the coastal State].