MOD	IRN/32/104

RESOLUTION  77  (Rev.WRC-03)

Protection of terrestrial services in all Regions from geostationary-satellite networks in the fixed-satellite service in Region 2 
using the frequency band 11.7-12.2 GHz

The World Radiocommunication Conference (Geneva, 2003),

considering

a)	that, in Regions 1 and 3, the band 11.7-12.2 GHz is allocated on a co-primary basis to terrestrial services and to the broadcasting-satellite service (BSS);

b)	that, in Region 2, the band 11.7-12.1 GHz is allocated on a co-primary basis to terrestrial services (except in the countries listed in No. 5.486) and to the fixed-satellite service (FSS);

c)	that, in Region 2, the band 12.1-12.2 GHz is allocated on a co-primary basis to terrestrial services in Peru (see No. 5.489) and to the FSS;

d)	that protection of the BSS in Regions 1 and 3 from the FSS in Region 2 is assured by Article 7 and Annex 4 of Appendix 30;

e)	that protection of the FSS in Region 2 from the FSS in that Region is assured either by Article 9 (Nos. 9.7 or 9.12) or Article 22;

f)	that protection of terrestrial services in Regions 1, 2 and 3 from non-geostationary- satellite (non-GSO) systems in the FSS in Region 2 is assured by Article 21;

g)	that there is a need to protect terrestrial services in Regions 1, 2 and 3 from GSO networks in the FSS in Region 2;

h) 	that the World Radiocommunication Conference 2000 (WRC-2000) has modified No. 5.488 by revising the regulatory limitations on the operation of GSO networks in the FSS in Region 2 in the band 11.7-12.2 GHz,

recognizing

that ITU-R has developed Recommendation ITU-R SF.674-1, dealing with sharing between the FSS and the fixed service in the band 11.7-12.2 GHz in Region 2,

resolves

1	that, before an administration notifies to the Radiocommunication Bureau or brings into use, in Region 2, a frequency assignment for a GSO network in the FSS in the 11.7-12.2 GHz band, it shall effect coordination pursuant to No. 9.14 of the Radio Regulations with any administration of Regions 1, 2, and 3 having a primary allocation to terrestrial services in the same frequency band if the power flux-density (pfd) produced on its territory exceeds the following thresholds:

–124 		dB(W/(m2 · MHz))	for 	0°   	5

–124  0.5 (– 5) 	dB(W/(m2 · MHz))	for 	5° <  	25

–114 		dB(W/(m2 · MHz))	for	         >	25

where  is the angle of arrival of the incident wave above the horizontal plane (degrees);1

2	that the above-mentioned procedure shall be applied by the Bureau for any submission received between 1 January 1999 until 3 June 2000,

instructs the Radiocommunication Bureau

1	in its examination of requests for coordination for any geostationary space station in the FSS operating in the band 11.7-12.2 GHz in Region 2, to determine if the pfd thresholds under resolves above are exceeded on the territory of any administration, other than the notifying administration, having a primary allocation to terrestrial services and, if so, to so notify both the notifying and the affected administrations;

2	to apply resolves 2 to frequency assignments received between 1 January 1999 until 3 June 2000 with respect to GSO FSS networks/systems referred to in resolves 1 above.

APPENDIX  5  (WRC-2000)

1	 	These values relate to the pfd and angles of arrival which would be obtained under free-space propagation conditions.