IRN/32/81

Method C as contained in Section 4.5.1.3.3 is supported, i.e. Resolution 128 would be suppressed and No. 5.551G would be modified to apply to the non-GSO in the band 42.0-42.5 GHz (as well as to GSO, as it currently does). The levels derived from the protection criteria in Recommendation ITU-R RA.769 would apply at radiotelescopes notified prior to the submission of advance publication information by the subject satellite network or system.

Resolution 84: Power flux-density limits in the bands 37.5-42.5 GHz for the fixed-satellite service, broadcasting-satellite service and mobile-satellite service.