ADD	USA/38/6

5.MSSDN	Additional allocation:  The band 1 430-1 432 MHz is also allocated on a primary basis to the mobile-satellite service (space-to-Earth), limited to feeder links for non-geostationary systems in the mobile-satellite service with service links below 1 GHz. In order to protect the radio astronomy service in the 1 400-1 427 MHz band, the aggregate power flux-density radiated by all space stations produced in the 1 400-1 427 MHz band shall not exceed the level of –180 dB(W/m2) in a 27 MHz band and a level of –196 dB(W/m2) in any 20 kHz portion of the 1 400-1 427 MHz band, at the site of any radio astronomy station for more than 2% of the time.

Reasons:	To provide additional allocations for non-GSO MSS feeder links (space-to-Earth) to support existing non-GSO MSS allocations below 1 GHz, and protect the radio astronomy service in the 1 400-1 427 MHz band.