ADD	CME/42/34

5.AAA	Mobile earth stations operating in the band 1 668-1 675 MHz shall not cause harmful interference to stations in the radio astronomy service operating in the band 1 660-1 670 MHz. The threshold levels of interference detrimental to the radio astronomy service are given in Recommendations ITU-R RA.769 and ITU-R RA.1513.

Reasons:	This method will provide protection to the existing services in the band. Few countries in fact use the 1 670-1 675 MHz band for MetAids operation; sharing with the MSS is a problem. Studies have shown that the spectrum requirements for MetAids could be satisfied in the 1 675-1 683 MHz band. Countries using the latter for MetAids operation could move to the 1 675-1 683 MHz band, leaving the 1 670-1 675 MHz band free for the MSS. The radioastronomy services in the 1 660-1 668 MHz band would be protected.