AFCP/57/38

The co-signatories in Table 1 for this agenda item support method A of the CPM Report. In this method for the 42-42.5 GHz band GSO network and 41.5-42.5 GHz band non-GSO systems, protect dish radio telescope sites on an individual, bilateral basis, modify levels in No. 5.551G to adopt the single dish, continuum threshold of –137 dB(W/(m2 · GHz)) and 153 dB(W/(m2 · 500 kHz)) as a threshold for bilateral arrangements as a single-dish radio telescope site.

A Resolution, which would not require the involvement of BR, would provide operational rules for bilateral sharing arrangement with affected single dish radio telescope sites where levels in No. 5.551G are exceeded for more than 2% of the time.

Reasons:	Studies conducted indicate that pfd levels produced by GSO satellite networks present no difficulty while the unwanted emission levels of the non-GSO FSS and BSS systems operating in the 42.0-43.5 GHz band met detrimental interference thresholds for the VLBI RAS stations operating in the 42.5-43.5 GHz band.