| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Addendum 2 to Document 64-E |
|  |  |  | 15 May 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| South Africa (Republic of) |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| agenda item 1.5 |  |

1.5	to consider, in accordance with Resolution 736 (WRC-2000), regulatory provisions and spectrum requirements for new and additional allocations to the mobile, fixed, Earth exploration-satellite and space research services, and to review the status of the radiolocation service in the frequency range 5 150-5 725 MHz, with a view to upgrading it, taking into account the results of ITU-R studies

# 1	Introduction

South Africa has identified radio local area networks (RLANs) as a key business driver in the development of broadband mobile wireless access in the future and therefore supports the allocation to mobile services, on a primary basis, of both the frequency bands 5 150-5 350 MHz and 5 470-5 725 MHz. South Africa also acknowledges the protection of the existing services and would therefore support the implementation of the necessary mitigation factors to ensure the protection of the existing services, without unduly constraining the development of RLANs in these frequency bands.

# 2	Proposals

ARTICLE  5

MOD	AFS/64A2/1

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 150-5 250 AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE ADD 5.RLAN 5.446 5.447B 5.447C |  |  |

5 250-5 255 EARTH EXPLORATION-SATELLITE (active) MOBILE ADD 5.RLAN RADIOLOCATION SPACE RESEARCH 5.447D 5.448 5.448A

5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) MOBILE ADD 5.RLAN RADIOLOCATION SPACE RESEARCH (active) 5.448 5.448A

MOD	AFS/64A2/2

5 350-5 460 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B AERONAUTICAL RADIONAVIGATION 5.449 RADIOLOCATION

MOD	AFS/64A2/3

5 460-5 470 EARTH EXPLORATION-SATELLITE (active) MOD 5.44B RADIONAVIGATION 5.449 RADIOLOCATION SPACE RESEARCH (active)

MOD	AFS/64A2/4

5 470-5 570 EARTH EXPLORATION-SATELLITE (active) MOD 5.44B MARITIME RADIONAVIGATION MOBILE ADD 5.RLAN RADIOLOCATION SPACE RESEARCH (active) 5.450 5.451 5.452

MOD	AFS/64A2/5

5 570-5 650 MARITIME RADIONAVIGATION MOBILE ADD 5.RLAN RADIOLOCATION 5.450 5.451 5.452

MOD	AFS/64A2/6

5 650-5 725 MOBILE ADD 5.RLAN RADIOLOCATION Amateur Space research (deep space) 5.282 5.451 5.453 5.454 5.455

Reasons: 	To make new primary allocations on a global basis to the mobile service in the frequency bands 5 150-5 350 MHz and 5 470-5 725 MHz, which will give the required regulatory status for mobile services. To facilitate sharing with the space services, sufficient spreading of WAS

including RLAN devices, are required, which requires that both the mentioned frequency bands be allocated to the mobile services, for use by WAS, including RLANs, at WRC-03. Having an allocation in both frequency bands will furthermore facilitate the implementation of DFS. Postponing the allocation to mobile for WAS, including RLANs, to WRC-07 in the upper band will unduly prejudice the mobile services if allocations to those other services considered under Resolution 736 (WRC-2000) are made by WRC-03.

The upgrade of the radiolocation service to primary in the band 5 350-5 650 MHz will provide the same regulatory status over the entire frequency range 5 250-5 725 MHz.

Primary allocations to both EESS (active) and SRS (active), as requested by resolves 3 of Resolution 736 (WRC-2000) in the frequency band 5 460-5 570 MHz is also supported.

ADD	AFS/64A2/7

5.RLAN	The allocation to the mobile service in the bands 5 150-5 350 MHz and 5 470-5 725 MHz is for the implementation of wireless access systems including radio local area networks, which will be operated in accordance with the following conditions:

–	in the 5 150-5 350 MHz band, the use of wireless access systems, including radio local area networks, shall be restricted to indoor use with a mean e.i.r.p. limit of 200 mW and a mean e.i.r.p. density limit no greater than 10 mW in any 1 MHz band;

–	in the band 5 470-5 725 MHz, the indoor and outdoor use of wireless access systems including radio local area networks, shall be restricted to a mean e.i.r.p. limit of 1 W and a mean e.i.r.p. density limit no greater than 50 mW in any 1 MHz band;

–	in the bands 5 250-5 350 MHz and 5 470-5 725 MHz, wireless access systems including radio local area networks, shall implement transmitter power control to ensure a mitigation factor of 3 dB on the mean output power. If transmitter power control is not implemented, the power limits given above shall be reduced by 3 dB;

–	in the bands 5 250-5 350 MHz and 5 470-5 725 MHz, wireless access systems, including radio local area networks, shall implement mitigation techniques such as dynamic frequency selection in accordance with [PDNR ITU-R M.[8A-9B.RLAN.DFS]] to avoid co-channel operation with other terrestrial systems operating in accordance with the table. These devices shall also be designed to provide a near uniform spread of the loading of the mobile stations across the available spectrum to improve sharing with satellite services.

Stations in the mobile service shall not claim protection from the fixed-satellite (Earth-to-space) service in the band 5 150-5 250 MHz or from the radiodetermination service in the bands 5 250-5 350 MHz and 5 470-5 725 MHz.

Reasons:	The specified operational restrictions, technical limits and mitigation techniques are to ensure protection for the fixed-satellite service (Earth-to-space) in the band 5 150-5 250 MHz, Earth exploration satellite-service (active) in the band 5 250-5 350 MHz and the radiolocation service in the bands 5 250-5 350 MHz and 5 470-5 725 MHz.

The inclusion of the text “WAS, including RLANs, must neither cause harmful interference to, nor claim protection from, nor otherwise impose constrains on operation or development of the existing services” as proposed in the CPM Report will place undue constraints in the future on the WAS, including RLAN services. Also, although it is accepted that WAS, including RLANs, cannot claim

protection from the existing primary allocations, the identified operational restrictions, technical limits and mitigation techniques to be imposed on the WAS, including RLANs, should afford the required protection to the other existing primary services.

A new Resolution for the continuation of studies on DFS could also be supported in case PDNR ITU-R M.[8A-9B.RLAN.DFS] has not been concluded in time for WRC-03.

MOD	AFS/64A2/8

5.448B	The earth exploration-satellite (active) service operating in the band 5 350-5 570 MHz shall not cause harmful interference to, or constrain the use and development of, the radionavigation services.   (WRC-03)

Reasons:	Sharing between EESS (active) and radionavigation in the bands 5 350-5 460 MHz and 5 460-5 570 MHz are identical, requiring an extension of the application of No. 5.448B.

SUP	AFS/64A2/9

5.447

Reasons:	This footnote is no longer required due to the global primary allocation to the mobile services.

_____________