ADD	AFS/64A2/7

5.RLAN	The allocation to the mobile service in the bands 5 150-5 350 MHz and 5 470-5 725 MHz is for the implementation of wireless access systems including radio local area networks, which will be operated in accordance with the following conditions:

–	in the 5 150-5 350 MHz band, the use of wireless access systems, including radio local area networks, shall be restricted to indoor use with a mean e.i.r.p. limit of 200 mW and a mean e.i.r.p. density limit no greater than 10 mW in any 1 MHz band;

–	in the band 5 470-5 725 MHz, the indoor and outdoor use of wireless access systems including radio local area networks, shall be restricted to a mean e.i.r.p. limit of 1 W and a mean e.i.r.p. density limit no greater than 50 mW in any 1 MHz band;

–	in the bands 5 250-5 350 MHz and 5 470-5 725 MHz, wireless access systems including radio local area networks, shall implement transmitter power control to ensure a mitigation factor of 3 dB on the mean output power. If transmitter power control is not implemented, the power limits given above shall be reduced by 3 dB;

–	in the bands 5 250-5 350 MHz and 5 470-5 725 MHz, wireless access systems, including radio local area networks, shall implement mitigation techniques such as dynamic frequency selection in accordance with [PDNR ITU-R M.[8A-9B.RLAN.DFS]] to avoid co-channel operation with other terrestrial systems operating in accordance with the table. These devices shall also be designed to provide a near uniform spread of the loading of the mobile stations across the available spectrum to improve sharing with satellite services.

Stations in the mobile service shall not claim protection from the fixed-satellite (Earth-to-space) service in the band 5 150-5 250 MHz or from the radiodetermination service in the bands 5 250-5 350 MHz and 5 470-5 725 MHz.

Reasons:	The specified operational restrictions, technical limits and mitigation techniques are to ensure protection for the fixed-satellite service (Earth-to-space) in the band 5 150-5 250 MHz, Earth exploration satellite-service (active) in the band 5 250-5 350 MHz and the radiolocation service in the bands 5 250-5 350 MHz and 5 470-5 725 MHz.