NOC	CTI/69/1

In view of the foregoing, the Administration of Côte d'Ivoire proposes not to make an allocation to the MSS in the bands 1 390-1 393 MHz and 1 429-1 432 MHz (methods A2 and B2 in the CPM Report).

Reasons:

1 390-1 393 MHz band

Protection of EESS (passive) in the band 1 400-1 427 MHz requires attenuation of unwanted emission levels of up to 128 dB on the MSS transmitter side. The technology for achieving these attenuation levels has not yet been tested. Use of the EESS (passive) secondary allocation in the band 1 370-1 400 MHz would be precluded due to in-band interference in areas where the MSS earth stations are within the field of view of EESS (passive) sensors.

In addition, continued access to the 1 330-1 400 MHz band by the RAS may be constrained if such an allocation is made.

1 429-1 452 MHz band

If an allocation is made, it would be difficult to protect the EESS (passive) in the 1 400-1 427 MHz band due to the impossibility of further reduction of available sensor bandwidth and insertion losses.

The band 1 427-1 452 MHz is used intensively for low-capacity long-haul radio relays and low-cost rural point-to-multipoint systems in Côte d’Ivoire.