| INTERNATIONAL TELECOMMUNICATION UNION |  |  |  |
| --- | --- | --- | --- |
|  | WRC-03 | WORLD RADIOCOMMUNICATION CONFERENCE | Document 78-E |
|  |  |  | 27 May 2003 |
|  |  |  | Original: English |
| GENEVA, 9 JUNE – 4 JULY 2003 |  |  |  |

|  | PLENARY MEETING |
| --- | --- |
| China (People's Republic of) |  |
| proposals for the work of the conference |  |
| agenda item 1.5 |  |

1.5	to consider, in accordance with Resolution 736 (WRC-2000), regulatory provisions and spectrum requirements for new and additional allocations to the mobile, fixed, Earth exploration-satellite and space research services, and to review the status of the radiolocation service in the frequency range 5 150-5 725 MHz, with a view to upgrading it, taking into account the results of ITU-R studies;

Introduction

Resolution 736 (WRC-2000) includes four resolves. China noted that the mobile service in the bands 5 150-5 350 MHz and 5 470-5 725 MHz for the implementation of WAS, including RLANs, can coexist with existing services in these bands by imposing appropriate technical restricts including DFS defined in draft new Recommendation ITU-R M.[8A-9B/RLAN-DFS]. When an FWA system and a radar station share the same frequency band, a certain geographical separation is necessary. Geographical separation is also needed for FWA system and RLANs to share the same frequency. The radiolocation service is widely used in the band 5 250-5 650 MHz in China, so coordination between FWA applications and RLS in that band will be very difficult. It is also difficult for FWA and wireless access systems, including RLANs, to operate on a co-frequency, co-location basis.

China basically supports Method A presented in Section 2.2.3.1.1 of the CPM Report for resolves 1, Method B in Section 2.2.3.2.2 for resolves 2, Method A in Section 2.2.3.3.1 for resolves 3 and Method A in Section 2.2.3.4.1 for resolves 4.

Proposals

article  5

MOD	CHN/78/1

4 800-5 830 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 150-5 250 AERONAUTICAL RADIONAVIGATION FIXED-SATELLITE (Earth-to-space) 5.447A MOBILE ADD 5.WAS1 5.446 5.447B 5.447C |  |  |

MOD	CHN/78/2

| 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.447D MOBILE ADD 5.WAS2 | 5 250-5 255 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH 5.447D MOBILE ADD 5.WAS2 |
| --- | --- |
| 5.448 | 5.448 ADD 5.FWA1 |

MOD	CHN/78/3

| 5 255- 5 350 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) MOBILE ADD 5.WAS2 | 5 255-5 350 EARTH EXPLORATION-SATELLITE (active) RADIOLOCATION SPACE RESEARCH (active) MOBILE ADD 5.WAS2 |
| --- | --- |
| 5.448 | 5.448 ADD 5.FWA1 |

MOD	CHN/78/4

5 350-5 460 EARTH EXPLORATION-SATELLITE (active) 5.448B AERONAUTICAL RADIONAVIGATION 5.449 RADIOLOCATION

MOD	CHN/78/5

5 460-5 470 RADIONAVIGATION 5.449 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B SPACE RESEARCH (active) ADD 5.SRS RADIOLOCATION

MOD	CHN/78/6

5 470-5 570 MARITIME RADIONAVIGATION MOBILE ADD 5.WAS3 EARTH EXPLORATION-SATELLITE (active) MOD 5.448B SPACE RESEARCH (active) ADD 5.SRS RADIOLOCATION 5.450 5.451 5.452

MOD	CHN/78/7

5 570-5 650 MARITIME RADIONAVIGATION MOBILE ADD 5.WAS3 RADIOLOCATION 5.450 5.451 5.452

MOD	CHN/78/8

5 650-5 725 RADIOLOCATION MOBILE ADD 5.WAS3 Amateur Space research (deep space) 5.282 5.451 5.453 5.454 5.455

Reasons:	5 150-5 350 MHz: China supports the new primary allocation to the mobile service in the band 5 150-5 350 MHz for the implementation of WAS (wireless access systems) including RLANs.

In order to ensure protection of the existing services, new footnotes specifying the operational and technical constraints for these new applications should be added. Considering some countries’ views of the new primary allocation to the fixed service in Region 3 in the band 5 250-5 350 MHz, China supports this new additional allocation to these countries in the footnote 5.FWA1.

5 350-5 725 MHz: China supports the new primary allocation to the mobile service in the band 5 470-5 725 MHz for the implementation of WAS (wireless access systems), including RLANs. In order to ensure protection of the existing services, a new footnote specifying the operational and technical constraints for this application should be added.

China also supports the primary allocations to the Earth exploration-satellite (active) and space research (active) services in the frequency band 5 460-5 570 MHz.

The constraints stated in the current footnote 5.448B for EESS (active) should be extended accordingly to the band 5 460-5 570 MHz, and the same kind of constraints should be specified also on the space research (active) service.

The radiolocation service in the band 5 350-5 650 MHz needs to be upgraded when the band is allocated to the mobile service on a primary basis, in order to provide the radiolocation service with an equal status to that of the mobile service.

ADD	CHN/78/9

5.WAS1	Use of the 5 150-5 250 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems, including radio local area networks, which shall be operated in accordance with the following conditions (see also Resolution [RLAN 2.3-1](WRC-03)):

–	deployment of wireless access systems, including radio local area networks, shall be restricted to indoor use only;

–	the mean equivalent isotropically radiated power density limit of any wireless access system station, including radio local area network stations, shall not exceed –20 dBW in any 1 MHz band (or equivalent –44 dBW in any 4 kHz band).

In the band 5 150-5 250 MHz, wireless access systems, including radio local area networks, must neither cause harmful interference to, nor claim protection from, nor otherwise impose constraints on operation or development of the existing services.

Reasons:	In order to protect feeder links (Earth-to-space) for non-GSO MSS and other existing services in this band, the above technical and operational restrictions should be applied to WAS including RLANs.

SUP	CHN/78/10

5.447

Reasons:	Suppression of No. 5.447 is conditional to the primary allocation of the band 5 150-5 250 MHz to the mobile service.

ADD	CHN/78/11

5.WAS2	Use of the 5 250-5 350 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems, including radio local area networks, which shall be operated in accordance with the following conditions:

–	deployment of wireless access systems, including radio local area networks, shall be restricted to indoor use only with a mean equivalent isotropically radiated power (e.i.r.p.) limit of –7 dBW and a mean e.i.r.p. density limit of 20 dBW in any 1 MHz band;

–	wireless access system stations, including radio local area network stations, shall implement transmitter power control to ensure a mitigation factor of at least 3 dB on the average output power. If transmitter power control is not implemented, the e.i.r.p. limits given above shall be reduced by 3 dB;

–	wireless access system stations, including radio local area network stations, shall implement dynamic frequency selection techniques described in [draft new] Recommendation ITU-R M.[8A-9B.RLAN.DFS] to avoid co-channel operation with other terrestrial systems, notably radar systems. The dynamic frequency selection devices shall also be designed to ensure a near uniform spread of the loading of the devices across the available spectrum to improve the sharing with satellite services.

In the band 5 250-5 350 MHz, wireless access systems, including radio local area networks, must neither cause harmful interference to, nor claim protection from, nor otherwise impose constraints on operation or development of the existing services.

Reasons:	In order to protect the existing services in the band 5 250-5 350 MHz the above technical and operational conditions should be applied to WAS, including RLANs.

ADD	CHN/78/12

5.FWA1	Additional allocation:  In [countries ...], the band 5 250-5 350 MHz is also allocated to the fixed service for fixed wireless access systems subject to compliance with Recommendation ITU-R F.[Doc.9/BL/46, FWA5GHz-EESS]. In addition, fixed wireless access systems shall not claim protection from the radiodetermination, Earth exploration-satellite (active) and space research (active) services, although the Earth exploration-satellite (active) and space research (active) services are not subject to No. 5.43A.

Reasons:	To enable operation of FWA systems in specific Region 3 countries while protecting the existing services in the band 5 250-5 350 MHz.

ADD	CHN/78/13

5.WAS3	Use of the 5 470-5 725 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems, including radio local area networks, which shall be operated in accordance with the conditions below:

–	the indoor and outdoor use of wireless access systems, including radio local area networks, shall be restricted to a mean e.i.r.p. limit of 0 dBW and a mean e.i.r.p. density limit of –13 dBW in any 1 MHz;

–	wireless access system stations, including radio local area network stations, shall implement transmitter power control to ensure a mitigation factor of at least 3 dB on the average output power. If transmitter power control is not implemented, the e.i.r.p. limits given above shall be reduced by 3 dB;

–	wireless access system stations, including radio local area network stations, shall implement dynamic frequency selection techniques described in [draft new] Recommendation ITU-R M.[8A-9B.RLAN.DFS] to avoid co-channel operation with other terrestrial systems, notably radar systems. The dynamic frequency selection devices shall also be designed to ensure a near uniform spread of the loading of the devices across the available spectrum to improve the sharing with satellite services.

In the band 5 450-5 725 MHz, wireless access systems, including radio local area networks, must neither cause harmful interference to, nor claim protection from, nor otherwise impose constraints on operation or development of the existing services.

Reasons:	The above technical and operational conditions should be applied to wireless access systems including radio local area networks in order to protect the existing services or other services to which new allocation is considered in the 5 470-5 725 MHz range.

SUP	CHN/78/14

5.448A

Reasons:	As operational experience shows the successful sharing between the EESS (active) and the RLS, the removal of No. 5.448A would not constrain the development and deployment of the radiolocation service.

MOD	CHN/78/15

5.448B	The earth exploration-satellite (active) service operating in the band 5 350-5 570 MHz shall not cause harmful interference to, or constrain the use and development of, the radionavigation service.   (WRC-03)

Reasons:	Consequential to extension of allocation for the Earth exploration-satellite (active) service to the band 5 460-5 570 MHz.

ADD	CHN/78/16

5.SRS	The space research (active) service operating in the band 5 460-5 570 MHz shall not cause harmful interference to, or constrain the use and development of, the radionavigation service.

Reasons:	For protection of the radionavigation service, the same constraints as those on Earth exploration-satellite (active) service should be specified also on the space research (active) service.

SUP	CHN/78/17

RESOLUTION  736  (WRC-2000)

Consideration by a future competent world radiocommunication conference
of issues dealing with allocations to the mobile, fixed, radiolocation,
Earth exploration-satellite (active), and space research (active)
services in the frequency range 5 150-5 725 MHz

Reasons:	All the resolves in Resolution 736 (WRC-2000) will be satisfied at WRC-03.

____________