ADD	CHN/78/11

5.WAS2	Use of the 5 250-5 350 MHz band by the mobile service is limited to those applications described in Recommendation ITU-R M.1450 for wireless access systems, including radio local area networks, which shall be operated in accordance with the following conditions:

–	deployment of wireless access systems, including radio local area networks, shall be restricted to indoor use only with a mean equivalent isotropically radiated power (e.i.r.p.) limit of –7 dBW and a mean e.i.r.p. density limit of 20 dBW in any 1 MHz band;

–	wireless access system stations, including radio local area network stations, shall implement transmitter power control to ensure a mitigation factor of at least 3 dB on the average output power. If transmitter power control is not implemented, the e.i.r.p. limits given above shall be reduced by 3 dB;

–	wireless access system stations, including radio local area network stations, shall implement dynamic frequency selection techniques described in [draft new] Recommendation ITU-R M.[8A-9B.RLAN.DFS] to avoid co-channel operation with other terrestrial systems, notably radar systems. The dynamic frequency selection devices shall also be designed to ensure a near uniform spread of the loading of the devices across the available spectrum to improve the sharing with satellite services.

In the band 5 250-5 350 MHz, wireless access systems, including radio local area networks, must neither cause harmful interference to, nor claim protection from, nor otherwise impose constraints on operation or development of the existing services.

Reasons:	In order to protect the existing services in the band 5 250-5 350 MHz the above technical and operational conditions should be applied to WAS, including RLANs.